# Project Button Independent Engineering Review

Complete Consolidated Handoff Dossier for Sol

Project Button HR-30-SYS-R0.2

6 August 2026

CONTROLLED STATUS: PRELIMINARY — NOT APPROVED FOR FABRICATION OR ENERGIZATION

## Handoff note to Sol

This is the single, self-contained engineering review package requested by Amy Sterling. It combines the full independent narrative assessment with every finding, calculation, requirement-audit record and unresolved electrical-selection record previously delivered as separate files.

Use the identifiers exactly as written. A finding is not closed by improving prose or changing a website label: closure requires the evidence stated in its record, linked to one immutable hardware/software configuration. Treat every BLOCKER as a prohibition on the affected release activity until formally dispositioned by the named qualified disciplines.

### Recommended reading order

- Executive decision and configuration contradictions.

- BLOCKER findings B-001 through B-018.

- Release-gate assessment and recommended engineering sequence.

- Appendix C for requirement rewrites and evidence gaps.

- Appendix D before any electrical procurement, panel wiring or harness fabrication.

# Independent Engineering Review — Project Button HR-30-SYS-R0.2

Review date: 2026-08-06
Authoritative repository revision reviewed: ee276af6f1a17c3a168f55efc91df2dd4a9eba38 (main)
Electrical identifier requested: Project Button Electrical V2.1
Reviewed hosted electrical artifacts: 15-page PDF, native netlist, connector schedule, wire table, unresolved-selection register, schematic BOM, electrical requirements trace, source register and ERC report
Release status: PRELIMINARY — NOT APPROVED FOR ENERGIZATION

Independent verdict: The package defines a coherent program architecture for a 762 mm humanoid, and HR-V0 is a plausible bench research mechanism. It does not yet define a buildable or energizable HR-V0, a detailed HR-30A, or a demonstrated walking drivetrain. The walking end-state remains physically plausible in principle, but the current package does not establish that the selected mass, actuators, power system, sensing and safety strategy close simultaneously.

## 1. Executive decision

| Assessment | Verdict | Basis |
| --- | --- | --- |
| HR-V0 build readiness | NOT READY | Architecture and requirements exist, but no buildable CAD, drawings, exact electrical selections, panel/harness definition, released procedures or implemented firmware. |
| HR-V0 energization readiness | PROHIBITED / NOT READY | The baseline itself forbids energization. Mains/PE/protection, safety-response time, contactor DC duty, watchdog fault tolerance and physical validation remain open. |
| HR-30A design readiness | SYSTEM CONCEPT ONLY | 13-axis inventory and release intent are coherent; detailed mechanical design, mass/thermal/power closure, pedestal, swept envelope and released electrical distribution do not exist. |
| HR-30W walking feasibility | PHYSICALLY PLAUSIBLE IN PRINCIPLE; NOT ESTABLISHED BY THIS DESIGN | The scale is feasible for humanoid research, but XH540+1.5:1 continuous/impact capability, mass/inertia, sensing, bus timing, power-loss behavior, battery/regen and dynamics are unproven. |
| Qualified mechanical review | READY FOR PDR / HAZARD REVIEW ONLY | There is enough system intent to challenge assumptions and define analyses; not enough drawings/calculations for final design review or fabrication release. |
| Qualified electrical review | READY FOR ARCHITECTURE REVIEW ONLY | Hosted PDF/netlist/exports support a preliminary topology review, but native source is absent from the authoritative repo and most physical selections/pins remain open. |
| Functional-safety review | READY TO START SRS/PLr WORK; NOT READY FOR VALIDATION | Hazards and a directionally sound E-stop concept exist, but no PLr/SIL allocation, architecture calculation, common-cause analysis or validated safety functions exist. |

### Finding count

- 18 BLOCKER findings

- 30 MAJOR findings

- 8 MINOR findings

- 62 requirements audited: 18 objective drafts with no evidence; 30 insufficient/unverifiable as written; remainder qualitative drafts with no evidence.

- Electrical exports: 142 tracked components, 103 selection-required; 168 named wire/net records; 106 unresolved-selection rows; all 10 electrical requirement-trace rows remain OPEN.

## 2. What is good and worth preserving

- The release ladder HR-V0 → HR-30A/B/C/D/W is the right systems-engineering instinct. It prevents a heroic leap from a rendering to a free-walking machine.

- The package repeatedly distinguishes stall torque from continuous torque and marks unresolved hardware as SELECTION REQUIRED. That candor is valuable.

- The E-stop concept includes two channels, monitored reset, two series contactors, external-device monitoring, separate ARM, and no automatic restart. Directionally, this is a strong starting architecture.

- The walking plan separates Linux behavior from real-time stabilization and hardware energy isolation, and W0-W5 includes test articles, restrained progression, fault injection and fall characterization.

- HR-V0’s actuator sizing is plausibly feasible: the independent gravity screen gives about 1.70 N·m shoulder and 0.62 N·m elbow static, versus official XM540 stall data of 10.6 N·m at 12 V. This is not proof of continuous life, but it is not fantasy.

- The repository openly labels the project adult-operated experimental machinery—not a toy—and blocks operation around children.

## 3. Review scope and limitations

This review challenged the design as machinery—not merely as a website, schematic drawing set, or configuration-management exercise. It included:

- Repository structure and release/gate logic.

- Mechanical gravity, dynamic-screening, support-polygon, fall-energy, restraint and mass checks.

- Official actuator torque/current/speed/voltage data and drivetrain implications.

- Native KiCad netlist topology plus exported BOM, wire, connector, selection, ERC and requirements records.

- Power-source, protection, grounding, regeneration and thermal assumptions.

- Walking timing, sensing, capture behavior, state estimation and safe-power-loss strategy.

- Control authority, reset/arm/enable/fault transitions and software evidence.

- Requirement-by-requirement verifiability and release inheritance.

Limitations are themselves material findings: the native KiCad source package was not present in the authoritative repository; no buildable CAD or firmware implementation was present; no physical test evidence exists. Therefore this is a preliminary design assurance review, not an approval, certification or final safety validation.

## 4. Independent calculations

All calculations below are screening calculations. They are not substitutes for released multibody, structural, thermal or protection analyses.

| Calculation | Result | Assumptions / formula |
| --- | --- | --- |
| HR-V0 shoulder static gravity torque | 1.702 N·m | Repo mass arms at horizontal |
| HR-V0 elbow static gravity torque | 0.620 N·m | Repo mass arms at horizontal |
| HR-V0 shoulder screening intermittent torque | 3.830 N·m | 1.5 acceleration × 1.5 uncertainty |
| HR-V0 elbow screening intermittent torque | 1.395 N·m | 1.5 acceleration × 1.5 uncertainty |
| Full-body fall potential energy — Target | 32.17 J | 8 kg × g × 0.41 m |
| Full-body fall potential energy — Hard limit | 45.11 J | 10 kg × g × 0.46 m |
| Static ankle moment, 8 kg, 40 mm COM offset | 3.138 N·m | Quasi-static whole-body screen |
| Static ankle moment, 8 kg, 150 mm COM offset | 11.77 N·m | Quasi-static whole-body screen |
| Static ankle moment, 10 kg, 40 mm COM offset | 3.923 N·m | Quasi-static whole-body screen |
| Static ankle moment, 10 kg, 150 mm COM offset | 14.71 N·m | Quasi-static whole-body screen |
| XH540 1.5:1 ideal reduced stall at 12 V | 14.85 N·m | No loss, zero-speed momentary stall |
| XH540 1.5:1 reduced stall with 85% assumed efficiency at 12 V | 12.62 N·m | Engineering screening assumption only |
| XH540 1.5:1 ideal reduced stall at 14.8 V | 17.55 N·m | No loss, zero-speed momentary stall |
| XH540 1.5:1 reduced stall with 85% assumed efficiency at 14.8 V | 14.92 N·m | Engineering screening assumption only |
| HR-V0 summed published stall current at 12 V | 11.10 A | 2× XM540 + 1× XM430 |
| HR-30A summed published stall current at 12 V | 42.80 A | 7× XM540 + 4× XM430 + 2× XC430 |
| Twelve XH540 leg actuators summed stall current at 14.8 V | 70.80 A | Does not include upper body |
| LIPM natural frequency at 0.41 m COM | 4.891 s⁻¹ | sqrt(g/z) |
| LIPM time constant at 0.41 m COM | 0.204 s | sqrt(z/g) |
| Capture-point offset at 0.10 m/s | 20.45 mm | v/omega |
| Capture-point offset at 0.20 m/s | 40.89 mm | v/omega |
| Fore-aft static tipping angle, 180 mm foot | 12.38 deg | half foot length / COM height |
| Lateral static tipping angle, 85 mm foot | 5.918 deg | half foot width / COM height |
| Average restraint force: 10 kg, 0.2 m drop, 0.05 m arrest | 490.3 N | mg + mgh/s = 5.0× weight average; peak higher |
| Average restraint force: 10 kg, 0.3 m drop, 0.05 m arrest | 686.5 N | mg + mgh/s = 7.0× weight average; peak higher |
| Average restraint force: 10 kg, 0.5 m drop, 0.05 m arrest | 1078.7 N | mg + mgh/s = 11.0× weight average; peak higher |
| One six-joint bus cycle serialization at 1 Mbit/s | 2.740 ms | 274 bytes assumed; excludes turnaround/retries |
| One six-joint bus cycle serialization at 3 Mbit/s | 0.913 ms | 274 bytes assumed; excludes turnaround/retries |
| One six-joint bus cycle serialization at 4.5 Mbit/s | 0.609 ms | 274 bytes assumed; excludes turnaround/retries |
| Default six-device return-delay overhead | 3.000 ms | ROBOTIS default 500 µs per device |
| 30 min battery energy at 100 W average | 50.00 Wh | Before reserve/conversion losses |
| 30 min battery capacity at 100 W average, 14.8 V | 3.378 Ah | Before reserve/conversion losses |
| 30 min battery energy at 150 W average | 75.00 Wh | Before reserve/conversion losses |
| 30 min battery capacity at 150 W average, 14.8 V | 5.068 Ah | Before reserve/conversion losses |
| 30 min battery energy at 250 W average | 125.0 Wh | Before reserve/conversion losses |
| 30 min battery capacity at 250 W average, 14.8 V | 8.446 Ah | Before reserve/conversion losses |
| Mass of 12 XH540 actuators | 1.980 kg | Manufacturer 165 g each |
| Remaining two-leg allocation after 12 XH540 actuators | 1.420 kg | For all reductions, bearings, shafts, links, feet, sensors, wiring and fasteners |
| Mass of four AK70-10 actuators | 2.084 kg | Hip pitch and knee on both legs |
| Mass increase of four AK70-10 vs four XH540 | 1.424 kg | Before power/bus/structure changes |

### 4.1 Interpretation of the torque screen

- The HR-V0 arm is plausible at concept level. The independent gravity calculation reproduces approximately 1.70 N·m shoulder and 0.62 N·m elbow static torque. Applying the package’s 2.25 screening multiplier gives 3.83 N·m and 1.39 N·m, below the XM540’s published momentary stall torque. Continuous temperature, gearbox life, side loads and hard-stop shocks still require tests.

- The leg screen is much less comfortable. An 8 kg body with COM only 40 mm from the ankle creates 3.14 N·m static; the package’s ×3 screen is 9.42 N·m. With an assumed 85% external-stage efficiency, XH540 at 14.8 V gives about 14.9 N·m momentary reduced stall. Using the manufacturer’s stall torque/current slope as a rough screen, 9.42 N·m would demand about 3.7 A, roughly 63% of stall current. A 10 kg, 150 mm-offset static case approaches the reduced-stall envelope before dynamics. This is not a continuous-duty design margin.

- The 1.5:1 stage multiplies motor reflected inertia by 2.25 at the joint before adding belt/pulley inertia and compliance. Rotor inertia and complete link inertias are absent, so control bandwidth and impact behavior cannot be evaluated.

### 4.2 Walking geometry interpretation

- A 180 × 85 mm foot has half-dimensions of 90 × 42.5 mm. At 0.41 m COM height, the static single-support tipping angles are only about 12.4° fore-aft and 5.9° lateral.

- The linear-inverted-pendulum time constant is about 0.204 s. At nominal 0.10-0.20 m/s, the velocity term alone corresponds to about 20-41 mm capture-point offset—roughly 23-45% of the fore-aft half-foot length before perturbation, latency, estimator error or safety margin.

- The geometry is not impossible; it simply demands a quantitative CoP/capture/saturation envelope that the current requirements do not provide.

### 4.3 Power and timing interpretation

- HR-V0’s 29 A source is not obviously undersized for its three candidates: their summed published 12 V stall current is about 11.1 A. The unresolved problems are protection coordination, DC interruption, regeneration, wiring and tested duty—not raw source nameplate power.

- HR-30A candidates sum to about 42.8 A published stall current at 12 V. A provisional 50 A source therefore needs a measured simultaneous-motion policy, converter transient model and fault/protection study.

- Twelve XH540 legs sum to 70.8 A published stall current at 14.8 V. An “80 A-class” tether does not, by itself, define safe selectivity, voltage stability or credible multi-axis saturation behavior.

- A representative six-joint Protocol 2.0 cycle with one 8-byte command and 20 bytes of telemetry per actuator serializes to roughly 2.74 ms at 1 Mbit/s or 0.91 ms at 3 Mbit/s, before turnaround, retries and diagnostics. Six default 500 µs return delays add 3 ms. The 250 Hz goal is credible only with a deliberately configured high-baud segmented bus and measured harness—not by assumption.

## 5. Configuration contradictions

| Topic | Document A | Document B / discrepancy |
| --- | --- | --- |
| Source-of-truth claim | README/configuration management say GitHub is authoritative; website says its exports are synchronized from the repository. | Current commit contains no native KiCad/CAD/firmware source. Hosted source package is outside the authoritative repo. |
| Electrical revision | Program calls package Project Button Electrical V2.1. | Hosted PDF title blocks say V2-P0.1; downloaded netlist says V2.1-P0.2. |
| Electrical counts | Website reports 136 components, 166 nets, 97 component selections open. | Downloaded current exports contain 142 components, 181 nets, 103 selection-required components and 106 unresolved-selection rows. |
| Program maturity counts | Website reports 50 draft requirements and 20 open risks. | Repository baseline reports 62 draft requirements and 40 open risks. |
| Electrical maturity | evidence-maturity.md calls HR-V0 connectivity E2 with connected KiCad V2.1. | electrical/README.md says no buildable schematic exists and the repo contains no ECAD source. |
| Actuator data | docs/mechanical.md associates XM540-W270-T with 9.9 N·m at 12 V. | Official ROBOTIS data give XM540 10.6 N·m at 4.4 A; 9.9 N·m at 4.9 A belongs to XH540. |
| Controller ownership | control.md says Raspberry Pi runs motion supervisor, trajectory executor and sensor processing. | walking-system.md assigns state estimation and stabilization to the real-time controller and says Linux cannot be the sole balance process. |
| Public rig disposition | independent-review-disposition.md says the running reference was removed. | Current public rig still lists a “Running reference” motion. |

## 6. Detailed findings

### 6.1 BLOCKER findings

#### B-001 — Authoritative repository does not contain the claimed native engineering implementation

Scope: All releases / configuration
Exact location: Repository tree at ee276af; electrical/README.md; cad/README.md; firmware/README.md; website Electrical §06
Exact item/net/component: electrical/ contains README.md only; cad/ contains README.md only; firmware/ contains README.md only
Risk: Independent reviewers cannot reproduce the KiCad/ERC result, inspect native source, build controlled CAD, or review implemented safety/control code from the stated source of truth. Matching-revision entry conditions cannot be satisfied.
Evidence required to close: Commit the exact KiCad .kicad_pro/.kicad_sch hierarchy, custom symbols/footprints, generated netlist/PDF/ERC artifacts and hashes; source CAD/STEP/drawings; watchdog/supervisor/RT firmware; tag a reviewed baseline.

#### B-002 — Electrical revision and metrics are internally inconsistent

Scope: All releases / configuration
Exact location: Hosted 15-page electrical PDF title blocks; downloaded native netlist; website Electrical §06; repository configuration-management.md
Exact item/net/component: PDF Rev V2-P0.1; netlist text variable/title blocks V2.1-P0.2; website says 136 components/166 nets/97 open selections and 50 requirements/20 risks; exports show 142 components/181 nets/103 selection-required components and repository says 62 requirements/40 risks
Risk: A reviewer cannot establish which configuration was checked. A clean ERC on one revision cannot support another revision, and test evidence could attach to the wrong design.
Evidence required to close: Issue one immutable electrical revision; regenerate PDF/netlist/BOM/wire/connector/selection/trace/manifest from that exact commit; publish generator version and SHA-256 manifest; update website counts automatically.

#### B-003 — No buildable HR-V0 mechanical definition exists

Scope: HR-V0 build
Exact location: cad/README.md; docs/mechanical.md; docs/releases/hr-v0.md; website HR-V0 assembly
Exact item/net/component: Bench mount, shafts, bearings, dual supports, hard stops, receiver fixture, guard, tolerances, mass properties, cable paths and drawings are absent
Risk: Parts cannot be fabricated, assembled, inspected, proof-loaded, or checked for interference safely. The 3D/mesh assets are fit-check or visual references only.
Evidence required to close: Release dimensioned CAD and 2D drawings with datums/GD&T or explicit tolerances, materials/heat treatment, fasteners/torques, bearing fits, hard-stop geometry, cable paths, guard and receiver fixture, plus signed calculation package.

#### B-004 — Hosted electrical package is architecture-level, not wireable

Scope: HR-V0 energization
Exact location: All 15 hosted KiCad sheets; electrical-bom.csv; connector-schedule.csv; wire-number-table.csv; unresolved-selections.csv
Exact item/net/component: 103 of 142 components marked SELECTION REQUIRED; 414 of 597 netlist endpoints use TBD terminal/function identifiers; F0-F3, S0/S1, QWD1, ISO1, enclosure, PE block and many terminals unresolved
Risk: Wire numbers and nets exist, but physical pins, ratings, protective devices, conductor sizes, connector keying, terminal blocks and installation details do not. Wiring from it can create shock, fire, backfeed and unsafe-motion faults.
Evidence required to close: Select exact MPNs and pin maps; release conductor/connector/fuse calculations and panel/harness drawings; eliminate every safety- or power-critical TBD; complete continuity/polarity/no-backfeed inspection before T2.

#### B-005 — Single non-safety watchdog relay contact can defeat heartbeat energy removal

Scope: HR-V0 and HR-30 safety function SAFE-003
Exact location: Sheet 02 refs KWD1/FSR1/SR1; sheet 04 refs WD1/QWD1; sheet 06 refs KWH1/FSRH1/SRH1; nets W2165-W2167 and W2075-W2077
Exact item/net/component: KWD1/KWH1 PLC-RSC-24DC/21 NO contact supplies both PNOZ output commons; no forced-guided monitoring or independent second watchdog channel is shown
Risk: A welded/stuck NO permit contact or driver fault can allow the safety relay outputs to remain eligible after heartbeat loss. The E-stop can still act, but SAFE-003’s claimed heartbeat-to-power-removal function is not fault tolerant or validated.
Evidence required to close: Either reclassify watchdog as a non-safety diagnostic and remove safety-performance claims, or design a monitored safety-rated architecture with an allocated PLr/SIL, diagnostic coverage, common-cause analysis, stuck-contact/brownout tests and independent latching.

#### B-006 — No safety-requirements specification or PLr/SIL allocation exists

Scope: Functional-safety review / all hazardous motion
Exact location: requirements.csv SAFE-001/003/008/009, WALK-012; risk-register.csv; docs/electrical.md; docs/control.md
Exact item/net/component: No safety-function list with initiating event, safe state, response time, demand rate, PLr/SIL target, architecture category, MTTFd/B10d, DCavg, CCF, systematic measures or validation plan
Risk: Component labels such as ‘safety relay’ do not establish safety performance. Common-cause and single-fault behavior cannot be judged, and a functional-safety reviewer has no normative design basis.
Evidence required to close: Create an ISO 12100 risk assessment and safety-requirements specification; choose ISO 13849-1/2 or IEC 62061 methodology; allocate PLr/SIL per function; calculate architecture and validate by analysis/test.

#### B-007 — Total safety response time and stopping distance are undefined

Scope: HR-V0 energization
Exact location: SAFE-003; docs/control.md watchdog 100 ms ×3; TEST-SAFE-001/002; sheets 02/04
Exact item/net/component: Requirement specifies 300 ms heartbeat loss only; it does not include watchdog detection/output, KWD1 release, PNOZ/contactors, supply discharge, servo bus watchdog, mechanical coast or residual travel
Risk: At the allowed 0.15 m/s TCP speed, 300 ms alone corresponds to 45 mm travel. Using typical relay/contactor delays raises the screening estimate to roughly 49-50 mm before servo coast and rail decay. Guard clearance cannot be approved from the current requirement.
Evidence required to close: Specify and test worst-case response time and stopping distance for each fault/pose/speed, including low/high voltage, temperature, suppression, contactor bounce, rail decay and actuator dynamics; design guard clearance to measured 95/99.7th-percentile plus tolerance.

#### B-008 — K1/K2 DC interruption suitability is not established

Scope: HR-V0 energization
Exact location: Sheet 02 refs K1/K2 LC1D25BD; nets V0_12V_FUSED, V0_K1_LOAD, V0_12V_SAFE; docs/electrical.md
Exact item/net/component: LC1D25BD is an AC-rated TeSys contactor candidate; application-specific 12 VDC inductive/regenerative interrupt current, arc behavior, pole use, suppression and life are not released
Risk: A contactor can weld, fail to interrupt regenerated current, or have unacceptable drop-out time even when its headline current exceeds load current.
Evidence required to close: Obtain manufacturer-approved DC utilization data for the exact wiring and fault current or select a documented DC contactor; calculate making/breaking duty and SCCR; validate repeated loaded interruption and welded-contact EDM faults.

#### B-009 — Protective-earth, DC-bonding, shielding and enclosure implementation are unresolved

Scope: HR-V0 mains energization
Exact location: Sheet 01 refs JAC1/Q0/QF1-QF3/PETB1/SP1/PS1-PS3; sheet 14 terminals; electrical.md
Exact item/net/component: SP1 DC 0V/PE bond is explicitly unpopulated pending review; inlet, PE block, branch protection, enclosure, shield treatment and terminal identifiers are selection required
Risk: Shock protection, fault return paths, EMC behavior and shared-impedance coupling cannot be verified. The package has no panel layout, bonding conductor design, creepage/clearance or PE continuity plan tied to exact hardware.
Evidence required to close: Release enclosure/panel layout, supply input protection, PE conductor/terminal/bonding design, single-point or isolated DC policy, shield termination by cable, prospective fault current/SCCR, and measured PE continuity/insulation tests.

#### B-010 — Full-body mass, center of mass and inertia are allocations, not a closed model

Scope: HR-30A through HR-30W
Exact location: docs/full-body-loads.md; MASS-001; G-20; evidence-maturity.md
Exact item/net/component: No controlled part-level mass ledger, link COMs, inertia tensors, cable/battery masses or reserve release; 8 kg is a target and 10 kg a ceiling
Risk: Torque, balance, energy, restraint, structural and controller calculations are all configuration-dependent. Treating the target as fact can undersize every downstream subsystem.
Evidence required to close: Release CAD and measured/CAD mass ledger by part/link, neutral and worst-pose COM, link and whole-body inertia tensors, 15-20% reserve policy, and configuration-specific simulation file.

#### B-011 — Provisional XH540 plus 1.5:1 drivetrain has no demonstrated continuous or impact margin

Scope: HR-30C/D/W standing and walking
Exact location: docs/walking-system.md Baseline A; docs/full-body-loads.md leg feasibility; WALK-BOM-001/002; XH540 manufacturer data
Exact item/net/component: 17.6 N·m is ideal zero-speed stall; no continuous torque curve, cyclic thermal limit, impact rating, shock load, efficiency or permissible output moment is established for the assembled joint
Risk: The screening model can consume 63-79% of stall current for selected ankle cases before dynamic disturbances. A robot may stand briefly yet overheat, saturate or damage gears/belts during gait or recovery.
Evidence required to close: Build instrumented pitch-joint and complete-leg articles; measure continuous/cyclic torque-speed-temperature, efficiency, backlash, compliance, shock/hard-stop response, gear/belt fatigue and current limits at 28 °C and worst supply.

#### B-012 — Leg drivetrain and structural load paths are not designed

Scope: HR-30B/C/D/W mechanical
Exact location: WALK-BOM-002/003; docs/walking-system.md; cad/README.md; W0/W1
Exact item/net/component: Pulley tooth counts/profile/width, belt tension, shafts, bearings, housings, fasteners, fits, output encoder mounts, hard stops, link sections and fatigue cases are all unresolved
Risk: The proposed reduction cannot be judged mechanically credible. Belt jump, pulley release, bearing overload, shaft bending, backlash and structural fatigue are unbounded.
Evidence required to close: Release joint CAD/drawings and tolerance stack; bearing/shaft/fastener/belt calculations for static, cyclic, impact and fall loads; FEA where appropriate; proof, endurance and slip/fault tests.

#### B-013 — Safe power-loss behavior is intentionally unresolved

Scope: HR-30W untethered walking
Exact location: SAFE-009; G-18/G-23; risks R-013/R-017/R-021/R-039; docs/walking-system.md Fall and stop behavior
Exact item/net/component: No brakes, passive knee geometry, counterbalance, retained control energy, latches or validated collapse strategy are selected
Risk: E-stop, undervoltage, contactor opening or controller loss can collapse the unbraked robot. A padded empty cell limits human exposure but does not establish a robust untethered machine architecture or protect the battery/head/hardware.
Evidence required to close: Choose and analyze a power-loss strategy per pose/mode; build representative joints/body; measure collapse trajectory, floor impact, containment and recovery; prove emergency energy removal remains effective.

#### B-014 — The fall-restraint requirement is not dynamically specified

Scope: HR-30B/C/D restraint
Exact location: PROD-004; TEST-RESTRAINT-001; HR-30B release; G-13; risk R-021/R-039
Exact item/net/component: Only a minimum static proof of 5× as-built weight is stated; no free-fall distance, arrest stroke, peak force, allowable robot acceleration, tether elasticity, rail/end-stop load, swing/pendulum clearance or attachment load distribution
Risk: A 0.2 m drop arrested in 0.05 m already requires an average load of about 5× weight; peak load is higher. Longer drops exceed the static proof basis. The restraint itself can injure/damage the robot or create a pendulum strike.
Evidence required to close: Define maximum drop geometry and arrest stroke; perform dynamic analysis with peak factors; rate every attachment/rail/harness; sacrificial-mass dynamic arrest tests; serialized inspection and retirement criteria.

#### B-015 — Battery, precharge, isolation, charger interlock and regeneration architecture are only functional blocks

Scope: HR-30W energy system
Exact location: Sheet 05; sheet 06 SRCSEL1/DC1/KH1/KH2; WALK-009; WALK-BOM-012/013; G-17
Exact item/net/component: Cell chemistry/capacity, BMS, pack fuse, contactors, precharge R/K, service disconnect, connector, charger interlock, telemetry, enclosure/venting and fault current are selection required
Risk: Runtime, fire/thermal propagation, short-circuit clearing, welded contact behavior, source paralleling and regeneration overvoltage cannot be evaluated.
Evidence required to close: Select qualified pack and charger; publish cell/pack limits, fault-current and arc analysis, BMS protections, fuse/contactor/precharge coordination, charge-drive hardware interlock, enclosure/venting and abuse/transport test plan.

#### B-016 — Foot-force and IMU electronics are not implementable circuits

Scope: HR-30C/D/W sensing
Exact location: Sheets 11-13; refs load cells, ADS131M04 blocks, IMU1/IMU2, sensor AFE; WALK-007; G-15
Exact item/net/component: Load-cell type/mount, excitation, bridge completion, reference, anti-alias filter, protection, ADC clocks/pins, PCB layout, synchronization, overload stops and calibration are unresolved
Risk: The claimed ±5 N/±5 mm and 500 Hz foot sensing, 800 Hz attitude data and saturation diagnostics cannot be shown. Sensor timing/grounding failures can directly destabilize the robot.
Evidence required to close: Release exact sensors, circuits/PCB/harness, timing and error budgets, mechanical overload design, calibration fixtures, temperature/drift/saturation tests and synchronized raw-data format.

#### B-017 — Real-time controller, firmware and bus timing are not implemented or demonstrated

Scope: HR-30C/D/W controls
Exact location: firmware/README.md; docs/control.md; docs/walking-system.md; WALK-014/G-22; sheets 04/07-12
Exact item/net/component: No RT code, scheduler, state estimator, stabilizer, bus drivers, HIL model, deadline monitor or packet-budget evidence is in the repository
Risk: The multi-rate schedule is an aspiration. Six-actuator half-duplex status traffic can consume most of a 4 ms cycle at 1 Mbit/s, especially with default return delays, retries and diagnostics.
Evidence required to close: Select controller/transceivers/baud/topology; byte-level budget with all fields; set and verify return delays; WCET/scheduling analysis; measured latency/jitter under retry storm; HIL and fault-injection logs.

#### B-018 — All requirements are draft and the governance requirement fails its own schema

Scope: Design-review release governance
Exact location: requirements/requirements.csv; safety/risk-register.csv; electrical-requirements-trace.csv; GOV-001/G-24
Exact item/net/component: 62/62 requirements are draft; 40/40 risks open; only 10 electrical requirements are traced and all are OPEN; requirement/risk tables omit owner, rationale, evidence URI, approver and change history required by GOV-001
Risk: No requirement can be declared passed, no residual risk is accepted by an accountable role, and later releases can attach evidence inconsistently.
Evidence required to close: Adopt controlled requirement/risk/gate schemas with owner, rationale, numeric acceptance, verification procedure, evidence artifact, independent approver and change record; baseline and approve before CDR/energization.

### 6.2 MAJOR findings

#### M-001 — Actuator data are cross-contaminated between XM540 and XH540

Scope: HR-V0 mechanical
Exact location: docs/mechanical.md vs BOM-005/006 and ROBOTIS official data
Exact item/net/component: Mechanical document states XM540-W270-T is 9.9 N·m at 12 V; official XM540 value is 10.6 N·m at 4.4 A, while 9.9 N·m is XH540 at 4.9 A
Risk: Incorrect model data propagate into current, torque and thermal calculations and indicate the manufacturer source was not tied to exact MPN.
Evidence required to close: Correct the actuator data table; store source revision/URL and extracted values per MPN; recalculate current/thermal/protection assumptions.

#### M-002 — Static torque multipliers do not replace dynamic and fatigue load cases

Scope: HR-V0 mechanical
Exact location: docs/mechanical.md worst-case static torque
Exact item/net/component: 1.5× acceleration ×1.5× uncertainty is applied without joint acceleration, link inertia, motor profile, backlash impact, hard-stop speed, cycle count or emergency-stop trajectory
Risk: Peak torque, shaft/bearing load and fatigue can be much larger or differently distributed than the scalar screen.
Evidence required to close: Define motion profiles/jerk and load spectrum; rigid/multibody dynamic model; hard-stop and dropped-payload cases; fatigue cycle target and Miner/S-N or test basis.

#### M-003 — Published actuator side-load limits are not reconciled to joint geometry

Scope: HR-V0/HR-30 joints
Exact location: XH/XM manufacturer data; docs/mechanical.md dual-support requirement
Exact item/net/component: XH540 publishes 40 N radial at 10 mm and 20 N axial; exact cantilever moments and bearing load sharing are not calculated
Risk: Actuator bearings/horns can be overloaded even when output torque is acceptable.
Evidence required to close: Calculate radial/axial/moment loads and shaft deflection at every joint; dual-supported bearing arrangement with fits/preload and proof test.

#### M-004 — Fastener, shaft, bearing, belt, pulley and structural calculations are absent

Scope: Mechanical all releases
Exact location: docs/mechanical.md ‘still required’; WALK-BOM; CAD release gap
Exact item/net/component: No bearing life, shaft combined stress, key/clamp friction, bolt preload/slip, thread engagement, plate bearing/tear-out, pulley flange, belt tooth/shear or base overturning calculations
Risk: Primary load paths and secondary retention cannot be independently reviewed or proof-tested to meaningful limits.
Evidence required to close: Released calculation notes with load cases, material allowables, safety factors, torque/preload specs and drawing references.

#### M-005 — Polymer material and process assumptions are unspecified

Scope: Printed parts / covers / gripper
Exact location: docs/mechanical.md; website experimental STL assets
Exact item/net/component: No resin/filament grade, print orientation, infill/perimeters, annealing, moisture, temperature, creep, impact, fatigue, aging or dimensional tolerance controls
Risk: Fit-check meshes may be mistaken for load-capable parts; anisotropy and creep can release guards, cables or gripper parts.
Evidence required to close: Material/process specification, coupon tests in all principal orientations, creep/heat/impact/fatigue criteria, print inspection and lot traceability; keep primary load paths metal until proven.

#### M-006 — Cable routing is a requirement without a geometrically verified design

Scope: Harness and serviceability
Exact location: dimension-control.md CAD deliverables; sheet 14; risk R-021/R-022/R-031/R-036
Exact item/net/component: No bend radii, cable stack, flex cycles, joint traversal, strain-relief loads, connector retention, shield terminations, service loops or tool/access envelopes
Risk: Cables can become hard stops, fatigue open, short, backfeed branches, snag the tether or be pinched by covers.
Evidence required to close: Harness CAD, cable specifications, bend/flex and strain tests, retention/secondary restraint, motion-cycle endurance and service procedure.

#### M-007 — Guard thickness, mounting and clearance are undefined

Scope: HR-V0 guard
Exact location: G-05; BOM-017; TEST-SAFE-001; TEST-MECH-001
Exact item/net/component: Clear polycarbonate is named but thickness, span, mounting, accessibility gaps, impact energy and residual stopping clearance are not specified
Risk: A guard can crack, detach or be entered through gaps; 600 mm floor marking does not replace fixed safeguarding.
Evidence required to close: Swept volume plus measured stop distance; material/thickness impact calculation/test; retained mounting; access-probe criteria; interlocked access if fixed-only access is impractical.

#### M-008 — The provisional 50 A source ceiling is not a demonstrated simultaneous-motion budget

Scope: HR-30A electrical
Exact location: HR30A BOM; website control/energy; G-12
Exact item/net/component: Seven XM540, four XM430 and two XC430 have about 42.8 A summed 12 V stall current using published values, before converter input current, margin and faults
Risk: A 50 A source may nuisance-trip or sag during credible multi-axis events, while protective devices may not clear individual faults selectively.
Evidence required to close: Measured trajectory current envelope, simultaneous joint policy, converter efficiency/transients, source droop, branch protection and contactor/conductor thermal coordination.

#### M-009 — 80 A-class tether source is not a protection or fault-current design

Scope: HR-30C/D power
Exact location: Sheet 06 JT1/FTHR1/FHR0; WALK-BOM-011
Exact item/net/component: Twelve XH540 leg actuators alone sum to 70.8 A published stall current at 14.8 V; upper-body converter load and regenerative transients are additional
Risk: Source current limit may interact with gait stability, branch fuses and contactors. A source ceiling is not selective branch protection.
Evidence required to close: Define credible simultaneous load/fault sets; source dynamic model; converter load; protection discrimination; bus voltage stability and injected current-limit tests.

#### M-010 — No closed system thermal model exists

Scope: Power and thermal
Exact location: THERM-001; TEST-THERM-001; power supplies/actuators/enclosures
Exact item/net/component: Ambient is defined, but airflow, enclosure volume, fan control, bundling, connector/PCB losses, converter efficiency, compute heat, thermal soak, duty cycle and derating are not
Risk: A 65 °C case limit and 20 °C wire rise cannot be extrapolated to 30-minute walking or enclosed body operation.
Evidence required to close: Loss budget by component and mode; thermal network/CFD as needed; worst ambient/blocked airflow tests; sensor accuracy/placement; derating and shutdown hysteresis.

#### M-011 — Regenerative energy has no selected sink or clamp

Scope: Actuator DC buses
Exact location: R-023; sheets 03/05/06; TEST-POWER-001; W3 fault injection
Exact item/net/component: Supplies, BMS and converters are not shown/qualified to absorb back-driven energy; no brake resistor, active clamp, dump load or rail capacitance calculation
Risk: Deceleration or collapse can overvoltage actuators/electronics, trip BMS, weld contacts or cause control loss.
Evidence required to close: Measure regenerative current/energy by motion and fall case; select bidirectional source or clamp/sink; overvoltage thresholds, thermal capacity and fault tests.

#### M-012 — Fuse discrimination and conductor sizing cannot be calculated from current inputs

Scope: Electrical protection
Exact location: QF1-QF3, F0-F3, FL/FR/FU branches; docs/electrical.md
Exact item/net/component: Prospective short-circuit current, source current-limit waveform, cable lengths/installation/bundling, insulation class, connector ratings and transient I²t are missing
Risk: Fuses may not clear, may nuisance-open during motion, or may fail to protect cable/connectors; upstream supply hiccup can defeat discrimination.
Evidence required to close: Measured/source-guaranteed fault current and time-current curves; conductor ampacity/voltage-drop/temperature calculations; fuse coordination study and fault injection.

#### M-013 — Custom data-only U2D2 harness is not released

Scope: DYNAMIXEL harness
Exact location: Sheet 03 UV1; sheets 09/10 UA1/UB1; docs/electrical.md; U2D2 pin 2
Exact item/net/component: VDD is intentionally omitted to avoid paralleled protected branches, but exact connectors, keying, ground continuity, shield, splice/breakout and no-backfeed validation are absent
Risk: A standard fully populated cable can silently bypass branch fuses or parallel rails.
Evidence required to close: Released custom harness drawing/MPNs, physical keying or blocked cavity, continuity/polarity/no-backfeed test and configuration inspection.

#### M-014 — Physical bus topology is not specified

Scope: RS-485 buses
Exact location: Sheets 07/08; G-22/WALK-014
Exact item/net/component: Transceiver, baud, termination locations/values, biasing, isolation, common-mode range, cable impedance/length, branch stubs, shield/ground, IDs and connector pinout remain open
Risk: A logically correct packet schedule can fail through reflections, common-mode currents, EMI or retry storms.
Evidence required to close: One-line and harness topology, exact transceivers/termination, signal-integrity review, scope captures under motor load, injected opens/shorts/termination faults.

#### M-015 — Direct-drive leg outputs lack independent output verification

Scope: Walking state estimation
Exact location: WALK-013 applies only to externally reduced joints; hip yaw/roll and ankle roll initially direct
Exact item/net/component: Motor encoder is upstream of internal gears/horn interface and cannot detect all gear/horn/backlash/output failures
Risk: A direct joint can report plausible motor position while the limb output slips or fractures.
Evidence required to close: Risk-based output sensing or mechanical integrity diagnostics for every balance-critical joint; proof that omitted sensors meet allocated safety/control requirements.

#### M-016 — Support-polygon margins are not quantified against dynamics

Scope: Foot geometry and stability
Exact location: Foot 180 × 85 mm; COM 0.41 m; walking-system performance
Exact item/net/component: Single-foot half-length is 90 mm and half-width 42.5 mm; no released CoP margin, ankle saturation margin, foot compliance model or allowed COM/capture-point region
Risk: The lateral static tipping angle is only about 5.9° at 0.41 m COM height. Small model errors, floor slope, sole deformation or sensor bias consume the margin.
Evidence required to close: Define safe CoP polygon inside the physical sole, load-dependent calibration region, torque/friction margins and simulation/test envelopes.

#### M-017 — Capture behavior and recovery authority are not bounded

Scope: Walking dynamics
Exact location: walking-system.md; W0-W5
Exact item/net/component: At COM height 0.41 m, LIPM time constant is about 0.204 s; nominal 0.1-0.2 m/s corresponds to 20-41 mm capture-point offset before disturbances, latency and model error
Risk: The specification has no push/recovery disturbance, maximum recoverable step, saturation or fall-declaration boundary.
Evidence required to close: Define disturbance set, capture-point/whole-body constraints, step timing and recovery thresholds; test progressively under tether with instrumented perturbations.

#### M-018 — Ground friction and step-width definitions are insufficient

Scope: Walking surface
Exact location: walking-system.md surface/step table; R-029
Exact item/net/component: No coefficient-of-friction range, contamination/wear condition, sole temperature, compliance, floor roughness or definition of step width (foot centers vs inner edges)
Risk: A gait may pass on one clean floor and fail on a nominally similar surface; acceptance cannot be reproduced.
Evidence required to close: Released surface coupons and friction test method/load/speed, sole wear/replacement limit, geometric definition and corridor survey.

#### M-019 — Controller responsibility is inconsistent across documents

Scope: Controls architecture
Exact location: docs/control.md Layering vs docs/walking-system.md real-time controller
Exact item/net/component: control.md assigns motion supervisor, trajectory executor and sensor processing to Raspberry Pi; walking-system.md assigns state estimation/stabilization to a real-time controller and says Linux cannot maintain balance
Risk: Authority, failure containment and software safety boundaries are ambiguous; implementation could accidentally place critical functions on Linux.
Evidence required to close: One canonical component/authority diagram and interface control document; assign every state, sensor, estimator, trajectory, watchdog and actuator register write to exactly one processor.

#### M-020 — Several fault responses are conditional but not testably bounded

Scope: Fault state machine
Exact location: docs/control.md fault table
Exact item/net/component: ‘controlled stop if safe’, ‘hold only within validated thermal/current window’, ‘removed when required’ and ‘applicable fault response’ have no algorithms, timers or thresholds
Risk: Different implementations or operators can choose different responses during the same hazard.
Evidence required to close: Fault-response matrix by state/pose/fault with deterministic transition, maximum time, energy state, braking/torque policy and fallback.

#### M-021 — AI prohibition is clear but command-source security is incomplete

Scope: Software permissions/security
Exact location: CTRL-001/002; docs/control.md
Exact item/net/component: No authenticated command channel, key management, update signing/rollback, debug-port policy, network segmentation or denial-of-service response is specified
Risk: A non-AI software or network fault could inject or sustain valid-looking commands.
Evidence required to close: Threat model tied to safety; authenticated signed release artifacts and commands, least privilege, secure boot/update strategy, network isolation and fuzz/failure tests.

#### M-022 — Evidence logging is specified but the timing/calibration infrastructure is absent

Scope: Logging/calibration
Exact location: CTRL-004/005; CFG-001/002; docs/control.md Logging
Exact item/net/component: No common clock architecture, synchronization error budget, immutable log format/schema, calibration procedure, scale accuracy, raw sensor units or retention policy
Risk: Fault reconstruction and cross-controller timing claims may be impossible even when data are recorded at nominal rates.
Evidence required to close: Clock source/synchronization design, timestamp uncertainty test, versioned log schema, calibration equipment/intervals and raw-record checksums.

#### M-023 — Critical qualification tests are missing

Scope: Verification program
Exact location: docs/verification.md and walking-verification.md
Exact item/net/component: No defined EMC immunity/emissions, ingress, vibration/transport, connector/cable flex, long-duration fatigue, maintenance error, hot-plug, misuse/abuse, drop/handling, battery propagation or environmental storage procedures
Risk: Release can pass scripted functional tests yet fail during normal handling, aging, maintenance or electromagnetic disturbance.
Evidence required to close: Add qualification matrix with severity-based applicability, fixtures, instrumentation, calibration, raw-data formats and independent sign-off.

#### M-024 — ‘Equivalent evidence’ can bypass HR-V0 without a controlled equivalence rule

Scope: Release inheritance
Exact location: docs/releases/hr-30a.md Entry conditions
Exact item/net/component: HR-30A permits HR-V0 acceptance ‘or every reused subsystem has equivalent evidence’ without defining equivalence authority, criteria or change-impact analysis
Risk: Later hardware can skip foundational safety/thermal/power tests by assertion.
Evidence required to close: Define mandatory inherited evidence, equivalence checklist, change-impact analysis and independent waiver authority; prohibit waiver of safety evidence without revalidation.

#### M-025 — Applicable product classification and jurisdiction remain open

Scope: Program/legal boundary
Exact location: G-19; full-body-specification.md
Exact item/net/component: Adult experimental machine is a project boundary, not a legal classification; country/site/user population and applicable electrical/battery/radio/product rules are not fixed
Risk: Procurement, enclosure, wiring, battery transport, EMC and public demonstration requirements can change materially by jurisdiction/intended use.
Evidence required to close: Document build/use jurisdiction and intended users; qualified legal/standards applicability assessment before procurement/public operation.

#### M-026 — The 3D rig cannot support packaging decisions

Scope: Packaging
Exact location: Website 3D Rig; evidence-maturity.md
Exact item/net/component: Exterior mesh has no verified cavity, wall thickness, structure, actuator envelopes, fasteners, harness, tool access, cooling or service clearance
Risk: Components can be selected around imaginary volume and later force mass/COM/thermal redesign.
Evidence required to close: Parametric mechanical CAD with vendor models, swept envelopes, cable and service volumes, collision/assembly study and mass properties.

#### M-027 — AK70-10 fallback carries a major mass/voltage/control penalty

Scope: Alternate walking actuator
Exact location: walking-system.md Baseline B; WALK-BOM-004
Exact item/net/component: Four hip-pitch/knee AK70 units weigh about 2.084 kg versus 0.660 kg for four XH540 units, adding about 1.424 kg before wiring or structural changes; AK70 is 24/48 V CAN versus the 14.8 V RS-485 baseline
Risk: The fallback can consume most mass reserve and creates a second power/bus/fault/service architecture.
Evidence required to close: System-level trade study with exact number of AK units, mass redistribution, power conversion, CAN timing, safety behavior, spares and performance evidence.

#### M-028 — Battery mass and runtime cannot close without measured average power

Scope: HR-30W battery budget
Exact location: full-body-loads.md 0.40 kg target/1.25 kg max; W5 30 min
Exact item/net/component: A 30-minute test needs 50 Wh at 100 W average, 75 Wh at 150 W and 125 Wh at 250 W before reserve and conversion losses
Risk: The aggressive 8 kg target may conflict with energy, current and enclosure requirements.
Evidence required to close: Measure gait power profile including peaks/regeneration; set reserve/end-of-life/cold-temperature policy; select a qualified pack and update mass/COM.

#### M-029 — Joint/output accuracy and saturation requirements are missing

Scope: Walking control
Exact location: walking-system.md sensing; W0-W5
Exact item/net/component: Sample rates are specified, but allowable joint position error, backlash, current/torque error, estimator covariance, actuator saturation duration and tracking-fault thresholds are not numerical
Risk: A high-rate system can still be too inaccurate or saturate before recovery.
Evidence required to close: Error budget from foot/COM requirements to encoder/actuator/estimator thresholds; measurable saturation and tracking limits.

#### M-030 — Unbraked arm fall and gravity behavior are not explicitly bounded

Scope: HR-V0/HR-30A power loss
Exact location: docs/control.md fault table; mechanical.md; HR-V0 guard concept
Exact item/net/component: Opening actuator power can let shoulder/elbow backdrive or drop a payload; no passive stop, counterbalance, catch, safe park geometry or measured drop envelope
Risk: Cat-0 removal may itself create impact/pinch inside the cell and determine required guard/receiver clearances.
Evidence required to close: Power-off backdrive tests at every released pose/payload; define mechanical catch/counterbalance or guard/catch-tray envelope; include in stop tests.

### 6.3 MINOR findings

#### N-001 — Website maturity counts are stale

Scope: Documentation
Exact location: Website lines 222-234 vs repository evidence-maturity.md
Exact item/net/component: Website shows 50 requirements/20 risks and 136 components/166 nets/97 selections; current artifacts show 62/40 and 142/181/103
Risk: Misleads reviewers about scope and undermines confidence in synchronization.
Evidence required to close: Generate website metrics from manifest-controlled artifacts.

#### N-002 — HR-30A required-evidence list duplicates INSPECT-PROD-002

Scope: Documentation
Exact location: docs/releases/hr-30a.md Required evidence
Exact item/net/component: INSPECT-PROD-001 through -003 already includes -002, then -002 is named again
Risk: Small traceability noise.
Evidence required to close: Remove duplicate and enumerate exact IDs.

#### N-003 — BMI088 description uses a qualitative ‘industrial-grade’ label

Scope: Sensor procurement
Exact location: WALK-BOM-005/006
Exact item/net/component: Label does not state required range, bandwidth, noise, bias instability, shock saturation or environmental qualification
Risk: Marketing shorthand can substitute for an engineering selection basis.
Evidence required to close: Replace with numeric sensor requirements and source-tested values.

#### N-004 — Several requirements combine multiple independent acceptance conditions

Scope: Requirements language
Exact location: WALK-002, WALK-004, CTRL-005, WALK-009, GOV-001
Exact item/net/component: One row bundles behavior, timing, configuration and safety outcomes
Risk: A row can partially pass and obscure which condition failed.
Evidence required to close: Split atomic requirements and retain parent trace links.

#### N-005 — Torque and rate notation is inconsistent

Scope: Units/notation
Exact location: Website/docs/BOM
Exact item/net/component: N m, N.m and N·m; deg/s and degrees; metres/m
Risk: Low technical risk but harms calculation transcription and machine parsing.
Evidence required to close: Adopt SI style guide and automated lint.

#### N-006 — Source-retrieval dates are not enough to freeze manufacturer evidence

Scope: Configuration
Exact location: references/primary-sources.md
Exact item/net/component: URLs and access dates are present, but exact PDFs/checksums are not stored in the repository baseline
Risk: Vendor pages can change after a review.
Evidence required to close: Archive permitted manufacturer PDFs or record document IDs/revisions and hashes in evidence package.

#### N-007 — Axis sign and coordinate conventions are incomplete

Scope: Walking geometry
Exact location: full-body-specification.md datums; walking-system.md joint ranges
Exact item/net/component: Global X is called sagittal center plane, but forward/lateral axes, positive rotations, left/right mirroring and zero conventions are not fully defined
Risk: Creates mirrored-joint polarity and simulation/CAD mismatch risk.
Evidence required to close: Release coordinate-frame standard and joint sign table with diagrams.

#### N-008 — Statistical treatment of endurance success is not defined

Scope: Test reporting
Exact location: VER-001 and W5 9/10 success criteria
Exact item/net/component: No confidence interval, failure classification, reset/rework rule or independence of trials
Risk: A small sample may be overinterpreted and reruns can hide failures.
Evidence required to close: Define trial independence, failure taxonomy, rerun policy and confidence/reporting method.

## 7. Requirement verification audit

The companion CSV audits all 62 requirements against three questions: (1) is there an objective verification method, (2) is the acceptance threshold sufficiently defined, and (3) can the proposed method actually demonstrate the requirement? No requirement has an executed, approved evidence record.

| audit_result | count |
| --- | --- |
| DRAFT — NO EVIDENCE | 14 |
| INSUFFICIENT / UNVERIFIABLE | 30 |
| OBJECTIVE DRAFT — NO EVIDENCE | 18 |

### Requirements most urgently requiring rewrite before test-procedure authoring

- SAFE-001/003/008/009 and WALK-012: convert into a safety-function specification with safe state, PLr/SIL, total response time, fault set and validation method.

- SAFE-006: define the actual operational, abnormal, impact and fatigue load cases; “3× maximum permitted static load” is currently circular.

- ELEC-003: replace “rated for the verified load” with exact load/fault/ambient/installation inputs and protection thresholds.

- CTRL-003, WALK-008, WALK-011 and STANCE-003: replace “defined/released/safe response” with deterministic state-specific actions and maximum times.

- THERM-001: replace provisional actuator limits with released limits tied to exact devices and duty cycles.

- PROD-004: define dynamic arrest geometry and forces, not merely “prevents floor impact.”

- WALK-014: define numerical bus utilization, latency, jitter, retry and missed-frame thresholds.

- GOV-001: add the fields it demands to the actual requirement/risk/gate schemas.

Companion file: Project_Button_Requirement_Audit.csv

## 8. Unresolved selections and procurement release

The hosted electrical register contains 106 unresolved selection rows. This review classifies 63 as release blockers and 43 as major closure items. Procurement should remain prohibited for any item whose MPN, pin map, rating, environment, coordination and evidence are not frozen.

| sheet | components | selection_required | proposed |
| --- | --- | --- | --- |
| 01_ac_pe_power | 10 | 8 | 2 |
| 02_hrv0_safety | 9 | 5 | 4 |
| 03_hrv0_distribution | 8 | 4 | 4 |
| 04_control_watchdog | 7 | 7 | 0 |
| 05_hr30_battery_precharge | 11 | 11 | 0 |
| 06_hr30_safety_distribution | 14 | 12 | 2 |
| 07_left_leg_rs485 | 15 | 9 | 6 |
| 08_right_leg_rs485 | 15 | 9 | 6 |
| 09_upper_bus_a | 17 | 8 | 9 |
| 10_upper_bus_b | 11 | 5 | 6 |
| 11_sensors_sync | 8 | 8 | 0 |
| 12_sensor_adc_imu | 4 | 4 | 0 |
| 13_sensor_afe | 1 | 1 | 0 |
| 14_harness_terminals | 12 | 12 | 0 |

### Minimum closure record for each electrical selection

- Exact manufacturer, MPN, revision and approved substitute policy.

- Official datasheet/manual revision and archived evidence identifier.

- Physical pin/terminal map checked against the received device.

- Voltage/current/fault/thermal/environmental rating calculation.

- Schematic, harness, BOM and panel-layout revision linkage.

- Installation/torque/crimp/strain-relief instructions.

- Inspection or test record proving the application-specific behavior.

- Independent reviewer and approval/change record.

Companion file: Project_Button_Electrical_Selection_Closure.csv

## 9. Release-gate assessment

### HR-V0 build gate

Do not release fabrication. Close B-001 through B-004 and the HR-V0 portions of B-006 through B-009, then complete mechanical drawings/calculations, exact BOM, panel/harness drawings, software baseline and T0 review. The actuator concept itself is plausible; the missing piece is a controlled machine definition.

### HR-V0 first energization gate

Do not energize. Required sequence:

- Qualified electrical and functional-safety review of one frozen native KiCad revision.

- Exact mains/PE/protection/contactor/watchdog selections and panel construction.

- Power-off continuity, polarity, PE, insulation, connector/no-backfeed and contactor-EDM checks.

- T2 safety energization with all actuator branches disconnected.

- Stuck/welded/broken-conductor/brownout/reset/stale-command fault injection.

- Only then, one branch at a time under current-limited T3 characterization.

### HR-30A detailed-design gate

Close the full-body CAD, pedestal/pelvis load path, mass/COM/inertia ledger, 13-axis current/thermal model, 12 V converter, DC contactors, harness/service envelope and two-arm fault tests. HR-V0 evidence may be inherited only through a formal equivalence/change-impact process.

### HR-30B/C/D/W walking path

The W0-W5 ladder should be retained, but no bulk leg procurement should occur until one pitch-joint article and one complete leg demonstrate continuous/cyclic/impact/thermal performance. Before reducing restraint dependence, close safe power-loss behavior, dynamic restraint, output sensing, bus timing, foot sensing, mass/inertia, regeneration, battery and fall containment.

## 10. Recommended engineering sequence

- Freeze configuration: put all native sources in GitHub, reconcile V2 identifiers/counts and tag the baseline.

- Write the safety-requirements specification: ISO 12100 hazard analysis plus ISO 13849 or IEC 62061 safety-function allocation.

- Finish HR-V0 detail design: CAD, calculations, guard, panel, harness, exact selections and firmware.

- Conduct qualified PDRs: mechanical, electrical and functional safety; disposition every blocker.

- Run T1-T3: physical inspection, safety-only energization and single-joint characterization.

- Earn HR-V0: proof load, thermal, fault injection and 100-handoff evidence.

- Close HR-30 mass/packaging: detailed body CAD and part-level mass/inertia reserve.

- Build walking test articles: primary joint, foot sensor, bus harness and one complete leg.

- Proceed through W1-W5 without waivers: each later configuration inherits exact earlier evidence and triggers change-impact review.

## 11. Final professional assessment

Project Button is not ready to build or energize today, but it is also not a mirage. The staged architecture, explicit unknowns and safety-minded gates are a strong foundation. HR-V0 is the correct next physical target and appears technically achievable after detail design. HR-30A is a credible concept. HR-30W is an ambitious research program whose feasibility remains conditional—not yet a design conclusion.

The single most important correction is to stop treating the hosted electrical model, visual rig and requirements prose as adjacent artifacts and turn them into one immutable, native, reviewable configuration. Once that spine exists, the remaining dragons are ordinary engineering dragons: torque, heat, wire, friction, timing and gravity. Formidable, yes. Magical, no.

## 12. Primary manufacturer and standards references

| Reference | Revision/date | URL | Use in review |
| --- | --- | --- | --- |
| ROBOTIS XH540-W270-T/R e-Manual | current page checked 2026-08-06 | https://emanual.robotis.com/docs/en/dxl/x/xh540-w270/ | 165 g; 272.5:1; 11.7 N·m/5.9 A stall at 14.8 V; 46 rpm; recommended 12 V; 0.25° backlash; explicit stall-vs-continuous warning. |
| ROBOTIS DYNAMIXEL-X model table | current page checked 2026-08-06 | https://emanual.robotis.com/docs/en/dxl/x/ | XM540/XM430/XC430 comparative torque, current, speed and voltage data. |
| ROBOTIS U2D2 Power Hub | current page checked 2026-08-06 | https://emanual.robotis.com/docs/en/parts/interface/u2d2_power_hub/ | 3.5-24.0 V, 10.0 A maximum. |
| CubeMars AK70-10 KV100 | current product page checked 2026-08-06 | https://www.cubemars.com/product/ak70-10-kv100-robotic-actuator.html | 521 g, 8.3 N·m rated, 24.8 N·m peak, 24/48 V architecture. |
| Pilz PNOZ s4 750104 | Operating Manual 21396-23, posted 2026-06-22 | https://www.pilz.com/en-US/eshop/product/750104 | 24 VDC; 3 safety NO/1 NC; two-channel and monitored-start capabilities; output ratings/application instructions. |
| Schneider TeSys LC1D25BD | official product data and linked-contact FAQ, FAQ modified 2026-05-12 | https://www.se.com/us/en/faqs/FA126437/ | LC1D family mirror/mechanically linked contacts; exact DC utilization suitability still application dependent. |
| Phoenix Contact PLC-RSC-24DC/21 | official product data checked 2026-08-06 | https://www.phoenixcontact.com/en-us/products/relay-module-plc-rsc-24dc21-c1d2-5603154 | General-purpose monostable relay; typical response/release timing; not itself a safety relay. |
| Mean Well LRS-350-12 | specification revision 2025-09-12 | https://www.meanwell.com/Upload/PDF/LRS-350/LRS-350-SPEC.PDF | 12 V, 29 A; overload/hiccup/peak and typical inrush data; component supply requiring final-system integration. |
| ISO 12100:2010 | confirmed current in 2022; revision underway | https://www.iso.org/standard/51528.html | Machinery risk assessment and risk reduction methodology. |
| ISO 13849-1:2023 | Edition 4, 2023-04 | https://www.iso.org/standard/73481.html | Design/integration of safety-related control systems and required performance levels. |
| ISO 13849-2:2012 | current published validation standard; successor draft underway | https://www.iso.org/standard/53640.html | Validation by analysis and test of safety functions/category/PL. |
| ISO 13850:2015 | Edition 3, 2015-11 | https://www.iso.org/standard/59970.html | Emergency-stop principles. |
| IEC 60204-1:2016+A1:2021 | valid consolidated Edition 6.1 | https://webstore.iec.ch/en/publication/71256 | Electrical equipment of machinery, protective bonding, overcurrent/SCCR, emergency stop and documentation. |
| IEC 62061:2021+A1:2024+A2:2026 | A2 published 2026-03-20 | https://webstore.iec.ch/en/publication/92835 | Functional safety of machinery safety-related control systems. |
| ISO 13482:2014 | current published service/personal-care robot standard; replacement at FDIS | https://www.iso.org/standard/53820.html | Relevant if the intended use becomes personal-care/service interaction; application must be determined by qualified review. |
| ISO/TR 23482-1:2020 | published 2020 | https://www.iso.org/standard/71564.html | Safety-related test methods supporting ISO 13482. |
| ISO 10218-1:2025 | Edition 3, 2025-02 | https://www.iso.org/standard/73933.html | Industrial-robot standard; explicitly excludes service robots/consumer/public-access uses, so applicability is limited. |

## 13. Embedded appendices

This consolidated dossier includes the complete contents of every companion register:

- Appendix A: all 56 BLOCKER, MAJOR and MINOR findings.

- Appendix B: all 40 independent calculations with assumptions and units.

- Appendix C: the complete 62-requirement verification audit.

- Appendix D: all 106 unresolved electrical selection-closure records.

- Appendix E: source-file inventory, row counts and SHA-256 integrity hashes.

No external spreadsheet is required to interpret this handoff.

This review is an independent engineering assessment, not a certification, legal opinion, stamped design, or authorization to fabricate or energize. Mains, machinery safety, batteries and full-height fall systems require qualified local professionals and application-specific validation.

# Appendix A — Complete Finding Register

Record count: 56 findings. The records below preserve every field from the original finding register.

A.1 — B-001: Authoritative repository does not contain the claimed native engineering implementation

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | All releases / configuration |
| Exact document / section / location | Repository tree at ee276af; electrical/README.md; cad/README.md; firmware/README.md; website Electrical §06 |
| Exact item / component / net | electrical/ contains README.md only; cad/ contains README.md only; firmware/ contains README.md only |
| Risk | Independent reviewers cannot reproduce the KiCad/ERC result, inspect native source, build controlled CAD, or review implemented safety/control code from the stated source of truth. Matching-revision entry conditions cannot be satisfied. |
| Evidence required to close | Commit the exact KiCad .kicad_pro/.kicad_sch hierarchy, custom symbols/footprints, generated netlist/PDF/ERC artifacts and hashes; source CAD/STEP/drawings; watchdog/supervisor/RT firmware; tag a reviewed baseline. |

A.2 — B-002: Electrical revision and metrics are internally inconsistent

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | All releases / configuration |
| Exact document / section / location | Hosted 15-page electrical PDF title blocks; downloaded native netlist; website Electrical §06; repository configuration-management.md |
| Exact item / component / net | PDF Rev V2-P0.1; netlist text variable/title blocks V2.1-P0.2; website says 136 components/166 nets/97 open selections and 50 requirements/20 risks; exports show 142 components/181 nets/103 selection-required components and repository says 62 requirements/40 risks |
| Risk | A reviewer cannot establish which configuration was checked. A clean ERC on one revision cannot support another revision, and test evidence could attach to the wrong design. |
| Evidence required to close | Issue one immutable electrical revision; regenerate PDF/netlist/BOM/wire/connector/selection/trace/manifest from that exact commit; publish generator version and SHA-256 manifest; update website counts automatically. |

A.3 — B-003: No buildable HR-V0 mechanical definition exists

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-V0 build |
| Exact document / section / location | cad/README.md; docs/mechanical.md; docs/releases/hr-v0.md; website HR-V0 assembly |
| Exact item / component / net | Bench mount, shafts, bearings, dual supports, hard stops, receiver fixture, guard, tolerances, mass properties, cable paths and drawings are absent |
| Risk | Parts cannot be fabricated, assembled, inspected, proof-loaded, or checked for interference safely. The 3D/mesh assets are fit-check or visual references only. |
| Evidence required to close | Release dimensioned CAD and 2D drawings with datums/GD&T or explicit tolerances, materials/heat treatment, fasteners/torques, bearing fits, hard-stop geometry, cable paths, guard and receiver fixture, plus signed calculation package. |

A.4 — B-004: Hosted electrical package is architecture-level, not wireable

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-V0 energization |
| Exact document / section / location | All 15 hosted KiCad sheets; electrical-bom.csv; connector-schedule.csv; wire-number-table.csv; unresolved-selections.csv |
| Exact item / component / net | 103 of 142 components marked SELECTION REQUIRED; 414 of 597 netlist endpoints use TBD terminal/function identifiers; F0-F3, S0/S1, QWD1, ISO1, enclosure, PE block and many terminals unresolved |
| Risk | Wire numbers and nets exist, but physical pins, ratings, protective devices, conductor sizes, connector keying, terminal blocks and installation details do not. Wiring from it can create shock, fire, backfeed and unsafe-motion faults. |
| Evidence required to close | Select exact MPNs and pin maps; release conductor/connector/fuse calculations and panel/harness drawings; eliminate every safety- or power-critical TBD; complete continuity/polarity/no-backfeed inspection before T2. |

A.5 — B-005: Single non-safety watchdog relay contact can defeat heartbeat energy removal

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-V0 and HR-30 safety function SAFE-003 |
| Exact document / section / location | Sheet 02 refs KWD1/FSR1/SR1; sheet 04 refs WD1/QWD1; sheet 06 refs KWH1/FSRH1/SRH1; nets W2165-W2167 and W2075-W2077 |
| Exact item / component / net | KWD1/KWH1 PLC-RSC-24DC/21 NO contact supplies both PNOZ output commons; no forced-guided monitoring or independent second watchdog channel is shown |
| Risk | A welded/stuck NO permit contact or driver fault can allow the safety relay outputs to remain eligible after heartbeat loss. The E-stop can still act, but SAFE-003’s claimed heartbeat-to-power-removal function is not fault tolerant or validated. |
| Evidence required to close | Either reclassify watchdog as a non-safety diagnostic and remove safety-performance claims, or design a monitored safety-rated architecture with an allocated PLr/SIL, diagnostic coverage, common-cause analysis, stuck-contact/brownout tests and independent latching. |

A.6 — B-006: No safety-requirements specification or PLr/SIL allocation exists

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | Functional-safety review / all hazardous motion |
| Exact document / section / location | requirements.csv SAFE-001/003/008/009, WALK-012; risk-register.csv; docs/electrical.md; docs/control.md |
| Exact item / component / net | No safety-function list with initiating event, safe state, response time, demand rate, PLr/SIL target, architecture category, MTTFd/B10d, DCavg, CCF, systematic measures or validation plan |
| Risk | Component labels such as ‘safety relay’ do not establish safety performance. Common-cause and single-fault behavior cannot be judged, and a functional-safety reviewer has no normative design basis. |
| Evidence required to close | Create an ISO 12100 risk assessment and safety-requirements specification; choose ISO 13849-1/2 or IEC 62061 methodology; allocate PLr/SIL per function; calculate architecture and validate by analysis/test. |

A.7 — B-007: Total safety response time and stopping distance are undefined

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-V0 energization |
| Exact document / section / location | SAFE-003; docs/control.md watchdog 100 ms ×3; TEST-SAFE-001/002; sheets 02/04 |
| Exact item / component / net | Requirement specifies 300 ms heartbeat loss only; it does not include watchdog detection/output, KWD1 release, PNOZ/contactors, supply discharge, servo bus watchdog, mechanical coast or residual travel |
| Risk | At the allowed 0.15 m/s TCP speed, 300 ms alone corresponds to 45 mm travel. Using typical relay/contactor delays raises the screening estimate to roughly 49-50 mm before servo coast and rail decay. Guard clearance cannot be approved from the current requirement. |
| Evidence required to close | Specify and test worst-case response time and stopping distance for each fault/pose/speed, including low/high voltage, temperature, suppression, contactor bounce, rail decay and actuator dynamics; design guard clearance to measured 95/99.7th-percentile plus tolerance. |

A.8 — B-008: K1/K2 DC interruption suitability is not established

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-V0 energization |
| Exact document / section / location | Sheet 02 refs K1/K2 LC1D25BD; nets V0_12V_FUSED, V0_K1_LOAD, V0_12V_SAFE; docs/electrical.md |
| Exact item / component / net | LC1D25BD is an AC-rated TeSys contactor candidate; application-specific 12 VDC inductive/regenerative interrupt current, arc behavior, pole use, suppression and life are not released |
| Risk | A contactor can weld, fail to interrupt regenerated current, or have unacceptable drop-out time even when its headline current exceeds load current. |
| Evidence required to close | Obtain manufacturer-approved DC utilization data for the exact wiring and fault current or select a documented DC contactor; calculate making/breaking duty and SCCR; validate repeated loaded interruption and welded-contact EDM faults. |

A.9 — B-009: Protective-earth, DC-bonding, shielding and enclosure implementation are unresolved

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-V0 mains energization |
| Exact document / section / location | Sheet 01 refs JAC1/Q0/QF1-QF3/PETB1/SP1/PS1-PS3; sheet 14 terminals; electrical.md |
| Exact item / component / net | SP1 DC 0V/PE bond is explicitly unpopulated pending review; inlet, PE block, branch protection, enclosure, shield treatment and terminal identifiers are selection required |
| Risk | Shock protection, fault return paths, EMC behavior and shared-impedance coupling cannot be verified. The package has no panel layout, bonding conductor design, creepage/clearance or PE continuity plan tied to exact hardware. |
| Evidence required to close | Release enclosure/panel layout, supply input protection, PE conductor/terminal/bonding design, single-point or isolated DC policy, shield termination by cable, prospective fault current/SCCR, and measured PE continuity/insulation tests. |

A.10 — B-010: Full-body mass, center of mass and inertia are allocations, not a closed model

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30A through HR-30W |
| Exact document / section / location | docs/full-body-loads.md; MASS-001; G-20; evidence-maturity.md |
| Exact item / component / net | No controlled part-level mass ledger, link COMs, inertia tensors, cable/battery masses or reserve release; 8 kg is a target and 10 kg a ceiling |
| Risk | Torque, balance, energy, restraint, structural and controller calculations are all configuration-dependent. Treating the target as fact can undersize every downstream subsystem. |
| Evidence required to close | Release CAD and measured/CAD mass ledger by part/link, neutral and worst-pose COM, link and whole-body inertia tensors, 15-20% reserve policy, and configuration-specific simulation file. |

A.11 — B-011: Provisional XH540 plus 1.5:1 drivetrain has no demonstrated continuous or impact margin

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30C/D/W standing and walking |
| Exact document / section / location | docs/walking-system.md Baseline A; docs/full-body-loads.md leg feasibility; WALK-BOM-001/002; XH540 manufacturer data |
| Exact item / component / net | 17.6 N·m is ideal zero-speed stall; no continuous torque curve, cyclic thermal limit, impact rating, shock load, efficiency or permissible output moment is established for the assembled joint |
| Risk | The screening model can consume 63-79% of stall current for selected ankle cases before dynamic disturbances. A robot may stand briefly yet overheat, saturate or damage gears/belts during gait or recovery. |
| Evidence required to close | Build instrumented pitch-joint and complete-leg articles; measure continuous/cyclic torque-speed-temperature, efficiency, backlash, compliance, shock/hard-stop response, gear/belt fatigue and current limits at 28 °C and worst supply. |

A.12 — B-012: Leg drivetrain and structural load paths are not designed

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30B/C/D/W mechanical |
| Exact document / section / location | WALK-BOM-002/003; docs/walking-system.md; cad/README.md; W0/W1 |
| Exact item / component / net | Pulley tooth counts/profile/width, belt tension, shafts, bearings, housings, fasteners, fits, output encoder mounts, hard stops, link sections and fatigue cases are all unresolved |
| Risk | The proposed reduction cannot be judged mechanically credible. Belt jump, pulley release, bearing overload, shaft bending, backlash and structural fatigue are unbounded. |
| Evidence required to close | Release joint CAD/drawings and tolerance stack; bearing/shaft/fastener/belt calculations for static, cyclic, impact and fall loads; FEA where appropriate; proof, endurance and slip/fault tests. |

A.13 — B-013: Safe power-loss behavior is intentionally unresolved

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30W untethered walking |
| Exact document / section / location | SAFE-009; G-18/G-23; risks R-013/R-017/R-021/R-039; docs/walking-system.md Fall and stop behavior |
| Exact item / component / net | No brakes, passive knee geometry, counterbalance, retained control energy, latches or validated collapse strategy are selected |
| Risk | E-stop, undervoltage, contactor opening or controller loss can collapse the unbraked robot. A padded empty cell limits human exposure but does not establish a robust untethered machine architecture or protect the battery/head/hardware. |
| Evidence required to close | Choose and analyze a power-loss strategy per pose/mode; build representative joints/body; measure collapse trajectory, floor impact, containment and recovery; prove emergency energy removal remains effective. |

A.14 — B-014: The fall-restraint requirement is not dynamically specified

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30B/C/D restraint |
| Exact document / section / location | PROD-004; TEST-RESTRAINT-001; HR-30B release; G-13; risk R-021/R-039 |
| Exact item / component / net | Only a minimum static proof of 5× as-built weight is stated; no free-fall distance, arrest stroke, peak force, allowable robot acceleration, tether elasticity, rail/end-stop load, swing/pendulum clearance or attachment load distribution |
| Risk | A 0.2 m drop arrested in 0.05 m already requires an average load of about 5× weight; peak load is higher. Longer drops exceed the static proof basis. The restraint itself can injure/damage the robot or create a pendulum strike. |
| Evidence required to close | Define maximum drop geometry and arrest stroke; perform dynamic analysis with peak factors; rate every attachment/rail/harness; sacrificial-mass dynamic arrest tests; serialized inspection and retirement criteria. |

A.15 — B-015: Battery, precharge, isolation, charger interlock and regeneration architecture are only functional blocks

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30W energy system |
| Exact document / section / location | Sheet 05; sheet 06 SRCSEL1/DC1/KH1/KH2; WALK-009; WALK-BOM-012/013; G-17 |
| Exact item / component / net | Cell chemistry/capacity, BMS, pack fuse, contactors, precharge R/K, service disconnect, connector, charger interlock, telemetry, enclosure/venting and fault current are selection required |
| Risk | Runtime, fire/thermal propagation, short-circuit clearing, welded contact behavior, source paralleling and regeneration overvoltage cannot be evaluated. |
| Evidence required to close | Select qualified pack and charger; publish cell/pack limits, fault-current and arc analysis, BMS protections, fuse/contactor/precharge coordination, charge-drive hardware interlock, enclosure/venting and abuse/transport test plan. |

A.16 — B-016: Foot-force and IMU electronics are not implementable circuits

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30C/D/W sensing |
| Exact document / section / location | Sheets 11-13; refs load cells, ADS131M04 blocks, IMU1/IMU2, sensor AFE; WALK-007; G-15 |
| Exact item / component / net | Load-cell type/mount, excitation, bridge completion, reference, anti-alias filter, protection, ADC clocks/pins, PCB layout, synchronization, overload stops and calibration are unresolved |
| Risk | The claimed ±5 N/±5 mm and 500 Hz foot sensing, 800 Hz attitude data and saturation diagnostics cannot be shown. Sensor timing/grounding failures can directly destabilize the robot. |
| Evidence required to close | Release exact sensors, circuits/PCB/harness, timing and error budgets, mechanical overload design, calibration fixtures, temperature/drift/saturation tests and synchronized raw-data format. |

A.17 — B-017: Real-time controller, firmware and bus timing are not implemented or demonstrated

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | HR-30C/D/W controls |
| Exact document / section / location | firmware/README.md; docs/control.md; docs/walking-system.md; WALK-014/G-22; sheets 04/07-12 |
| Exact item / component / net | No RT code, scheduler, state estimator, stabilizer, bus drivers, HIL model, deadline monitor or packet-budget evidence is in the repository |
| Risk | The multi-rate schedule is an aspiration. Six-actuator half-duplex status traffic can consume most of a 4 ms cycle at 1 Mbit/s, especially with default return delays, retries and diagnostics. |
| Evidence required to close | Select controller/transceivers/baud/topology; byte-level budget with all fields; set and verify return delays; WCET/scheduling analysis; measured latency/jitter under retry storm; HIL and fault-injection logs. |

A.18 — B-018: All requirements are draft and the governance requirement fails its own schema

| Field | Complete record |
| --- | --- |
| Severity | BLOCKER |
| Scope | Design-review release governance |
| Exact document / section / location | requirements/requirements.csv; safety/risk-register.csv; electrical-requirements-trace.csv; GOV-001/G-24 |
| Exact item / component / net | 62/62 requirements are draft; 40/40 risks open; only 10 electrical requirements are traced and all are OPEN; requirement/risk tables omit owner, rationale, evidence URI, approver and change history required by GOV-001 |
| Risk | No requirement can be declared passed, no residual risk is accepted by an accountable role, and later releases can attach evidence inconsistently. |
| Evidence required to close | Adopt controlled requirement/risk/gate schemas with owner, rationale, numeric acceptance, verification procedure, evidence artifact, independent approver and change record; baseline and approve before CDR/energization. |

A.19 — M-001: Actuator data are cross-contaminated between XM540 and XH540

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-V0 mechanical |
| Exact document / section / location | docs/mechanical.md vs BOM-005/006 and ROBOTIS official data |
| Exact item / component / net | Mechanical document states XM540-W270-T is 9.9 N·m at 12 V; official XM540 value is 10.6 N·m at 4.4 A, while 9.9 N·m is XH540 at 4.9 A |
| Risk | Incorrect model data propagate into current, torque and thermal calculations and indicate the manufacturer source was not tied to exact MPN. |
| Evidence required to close | Correct the actuator data table; store source revision/URL and extracted values per MPN; recalculate current/thermal/protection assumptions. |

A.20 — M-002: Static torque multipliers do not replace dynamic and fatigue load cases

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-V0 mechanical |
| Exact document / section / location | docs/mechanical.md worst-case static torque |
| Exact item / component / net | 1.5× acceleration ×1.5× uncertainty is applied without joint acceleration, link inertia, motor profile, backlash impact, hard-stop speed, cycle count or emergency-stop trajectory |
| Risk | Peak torque, shaft/bearing load and fatigue can be much larger or differently distributed than the scalar screen. |
| Evidence required to close | Define motion profiles/jerk and load spectrum; rigid/multibody dynamic model; hard-stop and dropped-payload cases; fatigue cycle target and Miner/S-N or test basis. |

A.21 — M-003: Published actuator side-load limits are not reconciled to joint geometry

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-V0/HR-30 joints |
| Exact document / section / location | XH/XM manufacturer data; docs/mechanical.md dual-support requirement |
| Exact item / component / net | XH540 publishes 40 N radial at 10 mm and 20 N axial; exact cantilever moments and bearing load sharing are not calculated |
| Risk | Actuator bearings/horns can be overloaded even when output torque is acceptable. |
| Evidence required to close | Calculate radial/axial/moment loads and shaft deflection at every joint; dual-supported bearing arrangement with fits/preload and proof test. |

A.22 — M-004: Fastener, shaft, bearing, belt, pulley and structural calculations are absent

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Mechanical all releases |
| Exact document / section / location | docs/mechanical.md ‘still required’; WALK-BOM; CAD release gap |
| Exact item / component / net | No bearing life, shaft combined stress, key/clamp friction, bolt preload/slip, thread engagement, plate bearing/tear-out, pulley flange, belt tooth/shear or base overturning calculations |
| Risk | Primary load paths and secondary retention cannot be independently reviewed or proof-tested to meaningful limits. |
| Evidence required to close | Released calculation notes with load cases, material allowables, safety factors, torque/preload specs and drawing references. |

A.23 — M-005: Polymer material and process assumptions are unspecified

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Printed parts / covers / gripper |
| Exact document / section / location | docs/mechanical.md; website experimental STL assets |
| Exact item / component / net | No resin/filament grade, print orientation, infill/perimeters, annealing, moisture, temperature, creep, impact, fatigue, aging or dimensional tolerance controls |
| Risk | Fit-check meshes may be mistaken for load-capable parts; anisotropy and creep can release guards, cables or gripper parts. |
| Evidence required to close | Material/process specification, coupon tests in all principal orientations, creep/heat/impact/fatigue criteria, print inspection and lot traceability; keep primary load paths metal until proven. |

A.24 — M-006: Cable routing is a requirement without a geometrically verified design

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Harness and serviceability |
| Exact document / section / location | dimension-control.md CAD deliverables; sheet 14; risk R-021/R-022/R-031/R-036 |
| Exact item / component / net | No bend radii, cable stack, flex cycles, joint traversal, strain-relief loads, connector retention, shield terminations, service loops or tool/access envelopes |
| Risk | Cables can become hard stops, fatigue open, short, backfeed branches, snag the tether or be pinched by covers. |
| Evidence required to close | Harness CAD, cable specifications, bend/flex and strain tests, retention/secondary restraint, motion-cycle endurance and service procedure. |

A.25 — M-007: Guard thickness, mounting and clearance are undefined

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-V0 guard |
| Exact document / section / location | G-05; BOM-017; TEST-SAFE-001; TEST-MECH-001 |
| Exact item / component / net | Clear polycarbonate is named but thickness, span, mounting, accessibility gaps, impact energy and residual stopping clearance are not specified |
| Risk | A guard can crack, detach or be entered through gaps; 600 mm floor marking does not replace fixed safeguarding. |
| Evidence required to close | Swept volume plus measured stop distance; material/thickness impact calculation/test; retained mounting; access-probe criteria; interlocked access if fixed-only access is impractical. |

A.26 — M-008: The provisional 50 A source ceiling is not a demonstrated simultaneous-motion budget

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-30A electrical |
| Exact document / section / location | HR30A BOM; website control/energy; G-12 |
| Exact item / component / net | Seven XM540, four XM430 and two XC430 have about 42.8 A summed 12 V stall current using published values, before converter input current, margin and faults |
| Risk | A 50 A source may nuisance-trip or sag during credible multi-axis events, while protective devices may not clear individual faults selectively. |
| Evidence required to close | Measured trajectory current envelope, simultaneous joint policy, converter efficiency/transients, source droop, branch protection and contactor/conductor thermal coordination. |

A.27 — M-009: 80 A-class tether source is not a protection or fault-current design

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-30C/D power |
| Exact document / section / location | Sheet 06 JT1/FTHR1/FHR0; WALK-BOM-011 |
| Exact item / component / net | Twelve XH540 leg actuators alone sum to 70.8 A published stall current at 14.8 V; upper-body converter load and regenerative transients are additional |
| Risk | Source current limit may interact with gait stability, branch fuses and contactors. A source ceiling is not selective branch protection. |
| Evidence required to close | Define credible simultaneous load/fault sets; source dynamic model; converter load; protection discrimination; bus voltage stability and injected current-limit tests. |

A.28 — M-010: No closed system thermal model exists

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Power and thermal |
| Exact document / section / location | THERM-001; TEST-THERM-001; power supplies/actuators/enclosures |
| Exact item / component / net | Ambient is defined, but airflow, enclosure volume, fan control, bundling, connector/PCB losses, converter efficiency, compute heat, thermal soak, duty cycle and derating are not |
| Risk | A 65 °C case limit and 20 °C wire rise cannot be extrapolated to 30-minute walking or enclosed body operation. |
| Evidence required to close | Loss budget by component and mode; thermal network/CFD as needed; worst ambient/blocked airflow tests; sensor accuracy/placement; derating and shutdown hysteresis. |

A.29 — M-011: Regenerative energy has no selected sink or clamp

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Actuator DC buses |
| Exact document / section / location | R-023; sheets 03/05/06; TEST-POWER-001; W3 fault injection |
| Exact item / component / net | Supplies, BMS and converters are not shown/qualified to absorb back-driven energy; no brake resistor, active clamp, dump load or rail capacitance calculation |
| Risk | Deceleration or collapse can overvoltage actuators/electronics, trip BMS, weld contacts or cause control loss. |
| Evidence required to close | Measure regenerative current/energy by motion and fall case; select bidirectional source or clamp/sink; overvoltage thresholds, thermal capacity and fault tests. |

A.30 — M-012: Fuse discrimination and conductor sizing cannot be calculated from current inputs

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Electrical protection |
| Exact document / section / location | QF1-QF3, F0-F3, FL/FR/FU branches; docs/electrical.md |
| Exact item / component / net | Prospective short-circuit current, source current-limit waveform, cable lengths/installation/bundling, insulation class, connector ratings and transient I²t are missing |
| Risk | Fuses may not clear, may nuisance-open during motion, or may fail to protect cable/connectors; upstream supply hiccup can defeat discrimination. |
| Evidence required to close | Measured/source-guaranteed fault current and time-current curves; conductor ampacity/voltage-drop/temperature calculations; fuse coordination study and fault injection. |

A.31 — M-013: Custom data-only U2D2 harness is not released

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | DYNAMIXEL harness |
| Exact document / section / location | Sheet 03 UV1; sheets 09/10 UA1/UB1; docs/electrical.md; U2D2 pin 2 |
| Exact item / component / net | VDD is intentionally omitted to avoid paralleled protected branches, but exact connectors, keying, ground continuity, shield, splice/breakout and no-backfeed validation are absent |
| Risk | A standard fully populated cable can silently bypass branch fuses or parallel rails. |
| Evidence required to close | Released custom harness drawing/MPNs, physical keying or blocked cavity, continuity/polarity/no-backfeed test and configuration inspection. |

A.32 — M-014: Physical bus topology is not specified

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | RS-485 buses |
| Exact document / section / location | Sheets 07/08; G-22/WALK-014 |
| Exact item / component / net | Transceiver, baud, termination locations/values, biasing, isolation, common-mode range, cable impedance/length, branch stubs, shield/ground, IDs and connector pinout remain open |
| Risk | A logically correct packet schedule can fail through reflections, common-mode currents, EMI or retry storms. |
| Evidence required to close | One-line and harness topology, exact transceivers/termination, signal-integrity review, scope captures under motor load, injected opens/shorts/termination faults. |

A.33 — M-015: Direct-drive leg outputs lack independent output verification

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Walking state estimation |
| Exact document / section / location | WALK-013 applies only to externally reduced joints; hip yaw/roll and ankle roll initially direct |
| Exact item / component / net | Motor encoder is upstream of internal gears/horn interface and cannot detect all gear/horn/backlash/output failures |
| Risk | A direct joint can report plausible motor position while the limb output slips or fractures. |
| Evidence required to close | Risk-based output sensing or mechanical integrity diagnostics for every balance-critical joint; proof that omitted sensors meet allocated safety/control requirements. |

A.34 — M-016: Support-polygon margins are not quantified against dynamics

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Foot geometry and stability |
| Exact document / section / location | Foot 180 × 85 mm; COM 0.41 m; walking-system performance |
| Exact item / component / net | Single-foot half-length is 90 mm and half-width 42.5 mm; no released CoP margin, ankle saturation margin, foot compliance model or allowed COM/capture-point region |
| Risk | The lateral static tipping angle is only about 5.9° at 0.41 m COM height. Small model errors, floor slope, sole deformation or sensor bias consume the margin. |
| Evidence required to close | Define safe CoP polygon inside the physical sole, load-dependent calibration region, torque/friction margins and simulation/test envelopes. |

A.35 — M-017: Capture behavior and recovery authority are not bounded

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Walking dynamics |
| Exact document / section / location | walking-system.md; W0-W5 |
| Exact item / component / net | At COM height 0.41 m, LIPM time constant is about 0.204 s; nominal 0.1-0.2 m/s corresponds to 20-41 mm capture-point offset before disturbances, latency and model error |
| Risk | The specification has no push/recovery disturbance, maximum recoverable step, saturation or fall-declaration boundary. |
| Evidence required to close | Define disturbance set, capture-point/whole-body constraints, step timing and recovery thresholds; test progressively under tether with instrumented perturbations. |

A.36 — M-018: Ground friction and step-width definitions are insufficient

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Walking surface |
| Exact document / section / location | walking-system.md surface/step table; R-029 |
| Exact item / component / net | No coefficient-of-friction range, contamination/wear condition, sole temperature, compliance, floor roughness or definition of step width (foot centers vs inner edges) |
| Risk | A gait may pass on one clean floor and fail on a nominally similar surface; acceptance cannot be reproduced. |
| Evidence required to close | Released surface coupons and friction test method/load/speed, sole wear/replacement limit, geometric definition and corridor survey. |

A.37 — M-019: Controller responsibility is inconsistent across documents

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Controls architecture |
| Exact document / section / location | docs/control.md Layering vs docs/walking-system.md real-time controller |
| Exact item / component / net | control.md assigns motion supervisor, trajectory executor and sensor processing to Raspberry Pi; walking-system.md assigns state estimation/stabilization to a real-time controller and says Linux cannot maintain balance |
| Risk | Authority, failure containment and software safety boundaries are ambiguous; implementation could accidentally place critical functions on Linux. |
| Evidence required to close | One canonical component/authority diagram and interface control document; assign every state, sensor, estimator, trajectory, watchdog and actuator register write to exactly one processor. |

A.38 — M-020: Several fault responses are conditional but not testably bounded

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Fault state machine |
| Exact document / section / location | docs/control.md fault table |
| Exact item / component / net | ‘controlled stop if safe’, ‘hold only within validated thermal/current window’, ‘removed when required’ and ‘applicable fault response’ have no algorithms, timers or thresholds |
| Risk | Different implementations or operators can choose different responses during the same hazard. |
| Evidence required to close | Fault-response matrix by state/pose/fault with deterministic transition, maximum time, energy state, braking/torque policy and fallback. |

A.39 — M-021: AI prohibition is clear but command-source security is incomplete

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Software permissions/security |
| Exact document / section / location | CTRL-001/002; docs/control.md |
| Exact item / component / net | No authenticated command channel, key management, update signing/rollback, debug-port policy, network segmentation or denial-of-service response is specified |
| Risk | A non-AI software or network fault could inject or sustain valid-looking commands. |
| Evidence required to close | Threat model tied to safety; authenticated signed release artifacts and commands, least privilege, secure boot/update strategy, network isolation and fuzz/failure tests. |

A.40 — M-022: Evidence logging is specified but the timing/calibration infrastructure is absent

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Logging/calibration |
| Exact document / section / location | CTRL-004/005; CFG-001/002; docs/control.md Logging |
| Exact item / component / net | No common clock architecture, synchronization error budget, immutable log format/schema, calibration procedure, scale accuracy, raw sensor units or retention policy |
| Risk | Fault reconstruction and cross-controller timing claims may be impossible even when data are recorded at nominal rates. |
| Evidence required to close | Clock source/synchronization design, timestamp uncertainty test, versioned log schema, calibration equipment/intervals and raw-record checksums. |

A.41 — M-023: Critical qualification tests are missing

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Verification program |
| Exact document / section / location | docs/verification.md and walking-verification.md |
| Exact item / component / net | No defined EMC immunity/emissions, ingress, vibration/transport, connector/cable flex, long-duration fatigue, maintenance error, hot-plug, misuse/abuse, drop/handling, battery propagation or environmental storage procedures |
| Risk | Release can pass scripted functional tests yet fail during normal handling, aging, maintenance or electromagnetic disturbance. |
| Evidence required to close | Add qualification matrix with severity-based applicability, fixtures, instrumentation, calibration, raw-data formats and independent sign-off. |

A.42 — M-024: ‘Equivalent evidence’ can bypass HR-V0 without a controlled equivalence rule

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Release inheritance |
| Exact document / section / location | docs/releases/hr-30a.md Entry conditions |
| Exact item / component / net | HR-30A permits HR-V0 acceptance ‘or every reused subsystem has equivalent evidence’ without defining equivalence authority, criteria or change-impact analysis |
| Risk | Later hardware can skip foundational safety/thermal/power tests by assertion. |
| Evidence required to close | Define mandatory inherited evidence, equivalence checklist, change-impact analysis and independent waiver authority; prohibit waiver of safety evidence without revalidation. |

A.43 — M-025: Applicable product classification and jurisdiction remain open

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Program/legal boundary |
| Exact document / section / location | G-19; full-body-specification.md |
| Exact item / component / net | Adult experimental machine is a project boundary, not a legal classification; country/site/user population and applicable electrical/battery/radio/product rules are not fixed |
| Risk | Procurement, enclosure, wiring, battery transport, EMC and public demonstration requirements can change materially by jurisdiction/intended use. |
| Evidence required to close | Document build/use jurisdiction and intended users; qualified legal/standards applicability assessment before procurement/public operation. |

A.44 — M-026: The 3D rig cannot support packaging decisions

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Packaging |
| Exact document / section / location | Website 3D Rig; evidence-maturity.md |
| Exact item / component / net | Exterior mesh has no verified cavity, wall thickness, structure, actuator envelopes, fasteners, harness, tool access, cooling or service clearance |
| Risk | Components can be selected around imaginary volume and later force mass/COM/thermal redesign. |
| Evidence required to close | Parametric mechanical CAD with vendor models, swept envelopes, cable and service volumes, collision/assembly study and mass properties. |

A.45 — M-027: AK70-10 fallback carries a major mass/voltage/control penalty

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Alternate walking actuator |
| Exact document / section / location | walking-system.md Baseline B; WALK-BOM-004 |
| Exact item / component / net | Four hip-pitch/knee AK70 units weigh about 2.084 kg versus 0.660 kg for four XH540 units, adding about 1.424 kg before wiring or structural changes; AK70 is 24/48 V CAN versus the 14.8 V RS-485 baseline |
| Risk | The fallback can consume most mass reserve and creates a second power/bus/fault/service architecture. |
| Evidence required to close | System-level trade study with exact number of AK units, mass redistribution, power conversion, CAN timing, safety behavior, spares and performance evidence. |

A.46 — M-028: Battery mass and runtime cannot close without measured average power

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-30W battery budget |
| Exact document / section / location | full-body-loads.md 0.40 kg target/1.25 kg max; W5 30 min |
| Exact item / component / net | A 30-minute test needs 50 Wh at 100 W average, 75 Wh at 150 W and 125 Wh at 250 W before reserve and conversion losses |
| Risk | The aggressive 8 kg target may conflict with energy, current and enclosure requirements. |
| Evidence required to close | Measure gait power profile including peaks/regeneration; set reserve/end-of-life/cold-temperature policy; select a qualified pack and update mass/COM. |

A.47 — M-029: Joint/output accuracy and saturation requirements are missing

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | Walking control |
| Exact document / section / location | walking-system.md sensing; W0-W5 |
| Exact item / component / net | Sample rates are specified, but allowable joint position error, backlash, current/torque error, estimator covariance, actuator saturation duration and tracking-fault thresholds are not numerical |
| Risk | A high-rate system can still be too inaccurate or saturate before recovery. |
| Evidence required to close | Error budget from foot/COM requirements to encoder/actuator/estimator thresholds; measurable saturation and tracking limits. |

A.48 — M-030: Unbraked arm fall and gravity behavior are not explicitly bounded

| Field | Complete record |
| --- | --- |
| Severity | MAJOR |
| Scope | HR-V0/HR-30A power loss |
| Exact document / section / location | docs/control.md fault table; mechanical.md; HR-V0 guard concept |
| Exact item / component / net | Opening actuator power can let shoulder/elbow backdrive or drop a payload; no passive stop, counterbalance, catch, safe park geometry or measured drop envelope |
| Risk | Cat-0 removal may itself create impact/pinch inside the cell and determine required guard/receiver clearances. |
| Evidence required to close | Power-off backdrive tests at every released pose/payload; define mechanical catch/counterbalance or guard/catch-tray envelope; include in stop tests. |

A.49 — N-001: Website maturity counts are stale

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Documentation |
| Exact document / section / location | Website lines 222-234 vs repository evidence-maturity.md |
| Exact item / component / net | Website shows 50 requirements/20 risks and 136 components/166 nets/97 selections; current artifacts show 62/40 and 142/181/103 |
| Risk | Misleads reviewers about scope and undermines confidence in synchronization. |
| Evidence required to close | Generate website metrics from manifest-controlled artifacts. |

A.50 — N-002: HR-30A required-evidence list duplicates INSPECT-PROD-002

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Documentation |
| Exact document / section / location | docs/releases/hr-30a.md Required evidence |
| Exact item / component / net | INSPECT-PROD-001 through -003 already includes -002, then -002 is named again |
| Risk | Small traceability noise. |
| Evidence required to close | Remove duplicate and enumerate exact IDs. |

A.51 — N-003: BMI088 description uses a qualitative ‘industrial-grade’ label

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Sensor procurement |
| Exact document / section / location | WALK-BOM-005/006 |
| Exact item / component / net | Label does not state required range, bandwidth, noise, bias instability, shock saturation or environmental qualification |
| Risk | Marketing shorthand can substitute for an engineering selection basis. |
| Evidence required to close | Replace with numeric sensor requirements and source-tested values. |

A.52 — N-004: Several requirements combine multiple independent acceptance conditions

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Requirements language |
| Exact document / section / location | WALK-002, WALK-004, CTRL-005, WALK-009, GOV-001 |
| Exact item / component / net | One row bundles behavior, timing, configuration and safety outcomes |
| Risk | A row can partially pass and obscure which condition failed. |
| Evidence required to close | Split atomic requirements and retain parent trace links. |

A.53 — N-005: Torque and rate notation is inconsistent

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Units/notation |
| Exact document / section / location | Website/docs/BOM |
| Exact item / component / net | N m, N.m and N·m; deg/s and degrees; metres/m |
| Risk | Low technical risk but harms calculation transcription and machine parsing. |
| Evidence required to close | Adopt SI style guide and automated lint. |

A.54 — N-006: Source-retrieval dates are not enough to freeze manufacturer evidence

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Configuration |
| Exact document / section / location | references/primary-sources.md |
| Exact item / component / net | URLs and access dates are present, but exact PDFs/checksums are not stored in the repository baseline |
| Risk | Vendor pages can change after a review. |
| Evidence required to close | Archive permitted manufacturer PDFs or record document IDs/revisions and hashes in evidence package. |

A.55 — N-007: Axis sign and coordinate conventions are incomplete

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Walking geometry |
| Exact document / section / location | full-body-specification.md datums; walking-system.md joint ranges |
| Exact item / component / net | Global X is called sagittal center plane, but forward/lateral axes, positive rotations, left/right mirroring and zero conventions are not fully defined |
| Risk | Creates mirrored-joint polarity and simulation/CAD mismatch risk. |
| Evidence required to close | Release coordinate-frame standard and joint sign table with diagrams. |

A.56 — N-008: Statistical treatment of endurance success is not defined

| Field | Complete record |
| --- | --- |
| Severity | MINOR |
| Scope | Test reporting |
| Exact document / section / location | VER-001 and W5 9/10 success criteria |
| Exact item / component / net | No confidence interval, failure classification, reset/rework rule or independence of trials |
| Risk | A small sample may be overinterpreted and reruns can hide failures. |
| Evidence required to close | Define trial independence, failure taxonomy, rerun policy and confidence/reporting method. |

# Appendix B — Complete Independent Calculation Register

Record count: 40 calculations. Values are screening calculations and retain their stated assumptions.

B.1 — HR-V0 shoulder static gravity torque

| Field | Complete record |
| --- | --- |
| Value | 1.70243444 |
| Unit | N·m |
| Assumptions or formula | Repo mass arms at horizontal |

B.2 — HR-V0 elbow static gravity torque

| Field | Complete record |
| --- | --- |
| Value | 0.6197802800000001 |
| Unit | N·m |
| Assumptions or formula | Repo mass arms at horizontal |

B.3 — HR-V0 shoulder screening intermittent torque

| Field | Complete record |
| --- | --- |
| Value | 3.83047749 |
| Unit | N·m |
| Assumptions or formula | 1.5 acceleration × 1.5 uncertainty |

B.4 — HR-V0 elbow screening intermittent torque

| Field | Complete record |
| --- | --- |
| Value | 1.3945056300000003 |
| Unit | N·m |
| Assumptions or formula | 1.5 acceleration × 1.5 uncertainty |

B.5 — Full-body fall potential energy — Target

| Field | Complete record |
| --- | --- |
| Value | 32.165811999999995 |
| Unit | J |
| Assumptions or formula | 8 kg × g × 0.41 m |

B.6 — Full-body fall potential energy — Hard limit

| Field | Complete record |
| --- | --- |
| Value | 45.110589999999995 |
| Unit | J |
| Assumptions or formula | 10 kg × g × 0.46 m |

B.7 — Static ankle moment, 8 kg, 40 mm COM offset

| Field | Complete record |
| --- | --- |
| Value | 3.138128 |
| Unit | N·m |
| Assumptions or formula | Quasi-static whole-body screen |

B.8 — Static ankle moment, 8 kg, 150 mm COM offset

| Field | Complete record |
| --- | --- |
| Value | 11.76798 |
| Unit | N·m |
| Assumptions or formula | Quasi-static whole-body screen |

B.9 — Static ankle moment, 10 kg, 40 mm COM offset

| Field | Complete record |
| --- | --- |
| Value | 3.9226599999999996 |
| Unit | N·m |
| Assumptions or formula | Quasi-static whole-body screen |

B.10 — Static ankle moment, 10 kg, 150 mm COM offset

| Field | Complete record |
| --- | --- |
| Value | 14.709974999999998 |
| Unit | N·m |
| Assumptions or formula | Quasi-static whole-body screen |

B.11 — XH540 1.5:1 ideal reduced stall at 12 V

| Field | Complete record |
| --- | --- |
| Value | 14.850000000000001 |
| Unit | N·m |
| Assumptions or formula | No loss, zero-speed momentary stall |

B.12 — XH540 1.5:1 reduced stall with 85% assumed efficiency at 12 V

| Field | Complete record |
| --- | --- |
| Value | 12.6225 |
| Unit | N·m |
| Assumptions or formula | Engineering screening assumption only |

B.13 — XH540 1.5:1 ideal reduced stall at 14.8 V

| Field | Complete record |
| --- | --- |
| Value | 17.549999999999997 |
| Unit | N·m |
| Assumptions or formula | No loss, zero-speed momentary stall |

B.14 — XH540 1.5:1 reduced stall with 85% assumed efficiency at 14.8 V

| Field | Complete record |
| --- | --- |
| Value | 14.917499999999997 |
| Unit | N·m |
| Assumptions or formula | Engineering screening assumption only |

B.15 — HR-V0 summed published stall current at 12 V

| Field | Complete record |
| --- | --- |
| Value | 11.100000000000001 |
| Unit | A |
| Assumptions or formula | 2× XM540 + 1× XM430 |

B.16 — HR-30A summed published stall current at 12 V

| Field | Complete record |
| --- | --- |
| Value | 42.8 |
| Unit | A |
| Assumptions or formula | 7× XM540 + 4× XM430 + 2× XC430 |

B.17 — Twelve XH540 leg actuators summed stall current at 14.8 V

| Field | Complete record |
| --- | --- |
| Value | 70.80000000000001 |
| Unit | A |
| Assumptions or formula | Does not include upper body |

B.18 — LIPM natural frequency at 0.41 m COM

| Field | Complete record |
| --- | --- |
| Value | 4.890670561036121 |
| Unit | s⁻¹ |
| Assumptions or formula | sqrt(g/z) |

B.19 — LIPM time constant at 0.41 m COM

| Field | Complete record |
| --- | --- |
| Value | 0.20447093860031812 |
| Unit | s |
| Assumptions or formula | sqrt(z/g) |

B.20 — Capture-point offset at 0.10 m/s

| Field | Complete record |
| --- | --- |
| Value | 20.447093860031814 |
| Unit | mm |
| Assumptions or formula | v/omega |

B.21 — Capture-point offset at 0.20 m/s

| Field | Complete record |
| --- | --- |
| Value | 40.89418772006363 |
| Unit | mm |
| Assumptions or formula | v/omega |

B.22 — Fore-aft static tipping angle, 180 mm foot

| Field | Complete record |
| --- | --- |
| Value | 12.380756928807177 |
| Unit | deg |
| Assumptions or formula | half foot length / COM height |

B.23 — Lateral static tipping angle, 85 mm foot

| Field | Complete record |
| --- | --- |
| Value | 5.918060352107626 |
| Unit | deg |
| Assumptions or formula | half foot width / COM height |

B.24 — Average restraint force: 10 kg, 0.2 m drop, 0.05 m arrest

| Field | Complete record |
| --- | --- |
| Value | 490.3325 |
| Unit | N |
| Assumptions or formula | mg + mgh/s = 5.0× weight average; peak higher |

B.25 — Average restraint force: 10 kg, 0.3 m drop, 0.05 m arrest

| Field | Complete record |
| --- | --- |
| Value | 686.4654999999999 |
| Unit | N |
| Assumptions or formula | mg + mgh/s = 7.0× weight average; peak higher |

B.26 — Average restraint force: 10 kg, 0.5 m drop, 0.05 m arrest

| Field | Complete record |
| --- | --- |
| Value | 1078.7314999999999 |
| Unit | N |
| Assumptions or formula | mg + mgh/s = 11.0× weight average; peak higher |

B.27 — One six-joint bus cycle serialization at 1 Mbit/s

| Field | Complete record |
| --- | --- |
| Value | 2.7399999999999998 |
| Unit | ms |
| Assumptions or formula | 274 bytes assumed; excludes turnaround/retries |

B.28 — One six-joint bus cycle serialization at 3 Mbit/s

| Field | Complete record |
| --- | --- |
| Value | 0.9133333333333333 |
| Unit | ms |
| Assumptions or formula | 274 bytes assumed; excludes turnaround/retries |

B.29 — One six-joint bus cycle serialization at 4.5 Mbit/s

| Field | Complete record |
| --- | --- |
| Value | 0.6088888888888888 |
| Unit | ms |
| Assumptions or formula | 274 bytes assumed; excludes turnaround/retries |

B.30 — Default six-device return-delay overhead

| Field | Complete record |
| --- | --- |
| Value | 3.0 |
| Unit | ms |
| Assumptions or formula | ROBOTIS default 500 µs per device |

B.31 — 30 min battery energy at 100 W average

| Field | Complete record |
| --- | --- |
| Value | 50.0 |
| Unit | Wh |
| Assumptions or formula | Before reserve/conversion losses |

B.32 — 30 min battery capacity at 100 W average, 14.8 V

| Field | Complete record |
| --- | --- |
| Value | 3.378378378378378 |
| Unit | Ah |
| Assumptions or formula | Before reserve/conversion losses |

B.33 — 30 min battery energy at 150 W average

| Field | Complete record |
| --- | --- |
| Value | 75.0 |
| Unit | Wh |
| Assumptions or formula | Before reserve/conversion losses |

B.34 — 30 min battery capacity at 150 W average, 14.8 V

| Field | Complete record |
| --- | --- |
| Value | 5.0675675675675675 |
| Unit | Ah |
| Assumptions or formula | Before reserve/conversion losses |

B.35 — 30 min battery energy at 250 W average

| Field | Complete record |
| --- | --- |
| Value | 125.0 |
| Unit | Wh |
| Assumptions or formula | Before reserve/conversion losses |

B.36 — 30 min battery capacity at 250 W average, 14.8 V

| Field | Complete record |
| --- | --- |
| Value | 8.445945945945946 |
| Unit | Ah |
| Assumptions or formula | Before reserve/conversion losses |

B.37 — Mass of 12 XH540 actuators

| Field | Complete record |
| --- | --- |
| Value | 1.98 |
| Unit | kg |
| Assumptions or formula | Manufacturer 165 g each |

B.38 — Remaining two-leg allocation after 12 XH540 actuators

| Field | Complete record |
| --- | --- |
| Value | 1.42 |
| Unit | kg |
| Assumptions or formula | For all reductions, bearings, shafts, links, feet, sensors, wiring and fasteners |

B.39 — Mass of four AK70-10 actuators

| Field | Complete record |
| --- | --- |
| Value | 2.084 |
| Unit | kg |
| Assumptions or formula | Hip pitch and knee on both legs |

B.40 — Mass increase of four AK70-10 vs four XH540

| Field | Complete record |
| --- | --- |
| Value | 1.424 |
| Unit | kg |
| Assumptions or formula | Before power/bus/structure changes |

# Appendix C — Complete Requirement Verification Audit

Record count: 62 requirements. Every original audit field is reproduced below.

## C.1 — System requirements

C.1 — SYS-001

| Field | Complete record |
| --- | --- |
| Level | system |
| Priority | MUST |
| Requirement statement | HR-V0 shall be a fixed bench-mounted mechanism with exactly two arm rotation axes and one gripper axis. |
| Verification ID | INSPECT-SYS-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Inspection ID has no released procedure or controlled CAD/BOM definition. |
| Evidence required to close | Release configuration reconciliation checklist and as-built count. |

C.2 — SYS-002

| Field | Complete record |
| --- | --- |
| Level | system |
| Priority | MUST |
| Requirement statement | HR-V0 shall transfer one soft foam object of no more than 100 g and 70 mm maximum dimension. |
| Verification ID | TEST-HAND-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | Numeric payload/object limits exist, but reference foam properties, fixture and dropped-object classification are not fixed. |
| Evidence required to close | Specify reference object, calibrated mass/dimensions, catch tray and trial-failure rules. |

C.3 — SYS-003

| Field | Complete record |
| --- | --- |
| Level | system |
| Priority | MUST |
| Requirement statement | Maximum shoulder-to-object reach shall not exceed 360 mm. |
| Verification ID | INSPECT-MECH-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | Reach threshold is numeric; datum, measurement pose and tolerance are not fully controlled without CAD. |
| Evidence required to close | Define shoulder/object datums, worst pose, instrument and tolerance. |

C.4 — SYS-004

| Field | Complete record |
| --- | --- |
| Level | system |
| Priority | MUST |
| Requirement statement | Automatic tool-center speed shall not exceed 0.15 m/s and joint speed shall not exceed 30 deg/s. |
| Verification ID | TEST-CTRL-001 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Joint/TCP speed limits are numeric; measurement bandwidth, filtering, transient overshoot and combined-axis TCP computation are absent. |
| Evidence required to close | Define calibrated measurement method, maximum transient/steady value and log computation. |

C.5 — SYS-005

| Field | Complete record |
| --- | --- |
| Level | system |
| Priority | MUST |
| Requirement statement | Setup motion shall be hold-to-run and shall not exceed 10 deg/s. |
| Verification ID | TEST-CTRL-002 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Speed is numeric; hold-to-run device, release latency and anti-tie-down behavior are unselected. |
| Evidence required to close | Select device and test release/dropout under all setup faults. |

C.6 — SYS-006

| Field | Complete record |
| --- | --- |
| Level | system |
| Priority | MUST |
| Requirement statement | The system shall operate indoors from 18 to 28 deg C in dry noncondensing conditions. |
| Verification ID | TEST-ENV-001 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Temperature range is numeric; humidity/noncondensing definition, stabilization time and storage/transport conditions are absent. |
| Evidence required to close | Define chamber/ambient measurement, soak, humidity and pass/fail behavior. |

## C.2 — Safety requirements

C.7 — SAFE-001

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | A dual-channel emergency stop shall remove actuator power without relying on software. |
| Verification ID | TEST-SAFE-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Method checks operation, but no PLr/SIL, response time, fault set, rail residual voltage or stopping threshold. |
| Evidence required to close | Create safety-function specification and ISO 13849/IEC 62061 validation plan. |

C.8 — SAFE-002

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | Releasing or resetting the emergency stop shall not initiate motion. |
| Verification ID | TEST-SAFE-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | No-motion intent is clear; allowable torque/current/position change after release/reset is not numeric. |
| Evidence required to close | Set zero-motion thresholds and test stale/nonzero command, boot and reset races. |

C.9 — SAFE-003

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | Loss of the supervisor heartbeat for 300 ms shall remove actuator power and latch a fault. |
| Verification ID | TEST-SAFE-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 300 ms is numeric, but total safety response and single-fault behavior of KWD1/QWD1 are not. |
| Evidence required to close | Specify total energy-removal/stopping time and validate stuck/welded/brownout faults. |

C.10 — SAFE-004

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | Fixed shielding and a controlled 600 mm exclusion boundary shall be present for powered V0 fixture tests. |
| Verification ID | INSPECT-SAFE-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 600 mm boundary is numeric; guard geometry/accessibility/stopping clearance and enforcement are not. |
| Evidence required to close | Release guard and exclusion-zone layout with access-probe and stop-distance evidence. |

C.11 — SAFE-005

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | No child shall enter the V0 test area while any actuator can be energized. |
| Verification ID | AUDIT-SAFE-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Administrative rule is auditable but no access-control mechanism, responsible person or violation response is defined. |
| Evidence required to close | Create controlled-area procedure, signage, lock/key and test-lead checklist. |

C.12 — SAFE-006

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | Primary mechanical load paths shall survive 3 times maximum permitted operational static load without hazardous failure. |
| Verification ID | TEST-MECH-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | 3× factor is numeric but ‘maximum permitted operational static load’ is undefined and excludes dynamic/fatigue/impact. |
| Evidence required to close | Release load cases and drawing tolerances; proof each load path with calibrated fixture. |

C.13 — SAFE-007

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | Each joint shall have independent software limits and mechanical hard stops before cable or shield contact. |
| Verification ID | INSPECT-MECH-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Presence can be inspected, but limits, stop energy, clearance and cable/guard margin are not numerical. |
| Evidence required to close | Define per-axis software/hardware limits and stop-clearance table from CAD. |

C.14 — SAFE-008

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | Any restart after a fault shall require cause removal, hardware reset, and a distinct software arm command. |
| Verification ID | TEST-SAFE-003 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | State sequence is conceptually testable; cause-removal/inspection criteria and timing are not formalized in code/procedure. |
| Evidence required to close | Release state-transition table and end-to-end reset/arm test matrix. |

## C.3 — Electrical requirements

C.15 — ELEC-001

| Field | Complete record |
| --- | --- |
| Level | electrical |
| Priority | MUST |
| Requirement statement | Actuator power, safety-control power, and compute power shall be separately protected domains. |
| Verification ID | INSPECT-ELEC-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Separate domains are visible, but ‘separately protected’ lacks exact devices, SCCR, selectivity and wiring. |
| Evidence required to close | Release protection coordination and panel/harness inspection procedure. |

C.16 — ELEC-002

| Field | Complete record |
| --- | --- |
| Level | electrical |
| Priority | MUST |
| Requirement statement | Two series contactors with monitored auxiliary feedback shall interrupt the actuator rail. |
| Verification ID | TEST-SAFE-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Topology exists; exact DC contactors and mirror behavior are unresolved, and no allocated safety performance exists. |
| Evidence required to close | Select devices, validate mirror contacts and inject welded/broken conductor faults. |

C.17 — ELEC-003

| Field | Complete record |
| --- | --- |
| Level | electrical |
| Priority | MUST |
| Requirement statement | Every actuator power branch shall be individually fused and use conductors and connectors rated for the verified load. |
| Verification ID | INSPECT-ELEC-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Circular phrase ‘rated for verified load’; load, fault current, cables and connector limits are unknown. |
| Evidence required to close | Complete current/fault/thermal/voltage-drop calculations and release exact parts. |

C.18 — ELEC-004

| Field | Complete record |
| --- | --- |
| Level | electrical |
| Priority | MUST |
| Requirement statement | Exposed mains terminals shall be inaccessible without a tool and conductive enclosures and structure shall be protectively earthed. |
| Verification ID | TEST-ELEC-001 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | General inspection/test is suitable, but enclosure/jurisdiction, PE path and dielectric thresholds are unselected. |
| Evidence required to close | Define jurisdictional test voltages, PE resistance/current, creepage and exact enclosure. |

## C.4 — Control requirements

C.19 — CTRL-001

| Field | Complete record |
| --- | --- |
| Level | control |
| Priority | MUST |
| Requirement statement | Only deterministic versioned trajectories whose preconditions match the current configuration may command motion. |
| Verification ID | TEST-CTRL-003 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | ‘Deterministic’ and all preconditions are not enumerated; signature/authentication and trajectory limits are unspecified. |
| Evidence required to close | Define command schema, validator, allowed trajectory set and negative tests. |

C.20 — CTRL-002

| Field | Complete record |
| --- | --- |
| Level | control |
| Priority | MUST |
| Requirement statement | AI or language-model output shall never write actuator commands or registers directly. |
| Verification ID | INSPECT-SW-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Code inspection can show direct access paths, but software architecture and code do not exist. |
| Evidence required to close | Implement privilege/API boundaries and static/runtime tests proving no alternate register path. |

C.21 — CTRL-003

| Field | Complete record |
| --- | --- |
| Level | control |
| Priority | MUST |
| Requirement statement | Communication loss, limit violation, overtemperature, or overcurrent shall cause a latched defined fault response. |
| Verification ID | TEST-FAULT-001 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | ‘Defined fault response’ is circular; response time, safe state and fault-specific thresholds are not in the requirement. |
| Evidence required to close | Split by fault and state with numerical detection/transition/energy limits. |

C.22 — CTRL-004

| Field | Complete record |
| --- | --- |
| Level | control |
| Priority | MUST |
| Requirement statement | Command and feedback state shall be logged at 100 Hz or faster with a configuration hash. |
| Verification ID | TEST-LOG-001 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | 100 Hz and hash are numeric; clock sync, fields, drop tolerance, immutability and retention are not. |
| Evidence required to close | Release log schema, timestamp error budget and completeness/checksum test. |

## C.5 — Thermal requirements

C.23 — THERM-001

| Field | Complete record |
| --- | --- |
| Level | thermal |
| Priority | MUST |
| Requirement statement | During the validated duty cycle actuator cases shall remain below provisional limits and power conductors below 20 deg C rise. |
| Verification ID | TEST-THERM-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | 20 °C conductor rise is numeric, but actuator limits are provisional and duty/measurement conditions are not final. |
| Evidence required to close | Replace provisional limits with released part/material limits and worst-case thermal procedure. |

## C.6 — Gripper requirements

C.24 — GRIP-001

| Field | Complete record |
| --- | --- |
| Level | gripper |
| Priority | MUST |
| Requirement statement | Grip force shall be characterized across the allowed object range using a calibrated load cell. |
| Verification ID | TEST-GRIP-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Characterization is not an acceptance requirement; no maximum/minimum force or object damage threshold. |
| Evidence required to close | Set force envelope, calibration accuracy, retention acceleration and pinch/contact limits. |

## C.7 — Verification requirements

C.25 — VER-001

| Field | Complete record |
| --- | --- |
| Level | verification |
| Priority | MUST |
| Requirement statement | The system shall complete 100 fixture handoffs with at least 99 successes and zero unsafe faults. |
| Verification ID | TEST-HAND-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 99/100 and zero unsafe faults are numeric; independence, rerun rules and unsafe-fault taxonomy are not. |
| Evidence required to close | Define trial protocol, failure classes, reset/rerun policy and statistical reporting. |

## C.8 — Configuration requirements

C.26 — CFG-001

| Field | Complete record |
| --- | --- |
| Level | configuration |
| Priority | MUST |
| Requirement statement | Every test record shall identify CAD, wiring, firmware, calibration, actuator, and BOM revisions. |
| Verification ID | AUDIT-CFG-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Audit fields are listed but format, evidence URI/hash, authority and mismatch handling are absent. |
| Evidence required to close | Release configuration-record schema and automated reconciliation. |

## C.9 — Product requirements

C.27 — PROD-001

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | HR-30 neutral-pose overall height shall be 762 mm nominal and remain between 740 and 800 mm for any released shell configuration. |
| Verification ID | INSPECT-PROD-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | Height thresholds are numeric; exact neutral pose, shell variants, floor datum and measurement uncertainty need procedure. |
| Evidence required to close | Release datum drawing and calibrated height inspection. |

C.28 — PROD-002

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | HR-30 tethered mass shall target 8.0 kg and shall never exceed 10.0 kg. |
| Verification ID | TEST-MASS-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 10 kg ceiling is objective; ‘target 8 kg’ is not pass/fail and tether/cable support conditions need definition. |
| Evidence required to close | Define weighed configuration and reserve disposition; calibrate scale. |

C.29 — PROD-003

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | HR-30A shall provide 13 active upper-body degrees of freedom on a bolted pelvic pedestal. |
| Verification ID | INSPECT-PROD-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Axis count is objective; actuator identity, lock state and allowed duplicate/auxiliary axes need controlled inventory. |
| Evidence required to close | Release exact joint inventory and configuration audit. |

C.30 — PROD-004

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | HR-30B full-height testing shall use an engineered restraint that prevents floor impact. |
| Verification ID | TEST-RESTRAINT-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | ‘Prevents floor impact’ is outcome-based but dynamic drop/arrest force/clearance are undefined. |
| Evidence required to close | Define drop envelope, arrest stroke/force and dynamic sacrificial-mass test. |

C.31 — PROD-005

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | The full-height robot shall remain restrained through HR-30D and shall operate untethered only after the HR-30W release gate is approved. |
| Verification ID | AUDIT-PROD-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Audit can enforce sequencing, but approval authority and immutable gate evidence are not defined. |
| Evidence required to close | Create signed gate ledger and interlock/config rule preventing unapproved untethered mode. |

C.32 — PROD-006

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | External covers shall use a 3 mm minimum edge radius and prevent normal access to powered pinch points. |
| Verification ID | INSPECT-PROD-003 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 3 mm radius is numeric; access probe set, gaps, force and every-pose coverage are unspecified. |
| Evidence required to close | Select accessibility standard/probes and inspect all configurations/poses. |

C.33 — PROD-007

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | SHOULD |
| Requirement statement | The external form shall be stylized and clearly robotic rather than a realistic imitation child. |
| Verification ID | REVIEW-ID-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Subjective design-review criterion has no rubric; acceptable because SHOULD but cannot be objectively passed as written. |
| Evidence required to close | Create visual/anthropomorphic design rubric and named approver. |

C.34 — PROD-008

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | HR-30A shall use tethered extra-low-voltage actuator power and shall not carry a battery. |
| Verification ID | INSPECT-ELEC-030 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Inspection is conceptually objective; ‘extra-low-voltage’ value, isolation and absence-of-energy-storage definition need exact architecture. |
| Evidence required to close | Specify rail limits, source and inspection of all stored-energy devices. |

C.35 — PROD-009

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | Actuator power state and faults shall be visibly distinguishable from at least 3 m. |
| Verification ID | TEST-UI-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 3 m is numeric; ambient lighting, viewing angle, color-independent encoding and recognition success threshold need final procedure. |
| Evidence required to close | Release lighting/viewing matrix and observer/photometric acceptance. |

C.36 — PROD-010

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | Camera recording capability shall have a hardware-driven visible privacy indication. |
| Verification ID | TEST-PRIV-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Hardware-driven intent is testable; indicator logic, visibility, failure state and camera power/capture boundary are undefined. |
| Evidence required to close | Define electrical truth table and injected open/short/boot/power faults. |

## C.10 — Legs requirements

C.37 — LEG-001

| Field | Complete record |
| --- | --- |
| Level | legs |
| Priority | MUST |
| Requirement statement | No powered leg joint shall be enabled until its actuator and structure pass separate load thermal brake and restraint tests. |
| Verification ID | AUDIT-LEG-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — concept method described |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Audit names broad evidence but load/thermal/brake/restraint thresholds are not specified per joint. |
| Evidence required to close | Create per-joint qualification specification and signed evidence checklist. |

C.38 — LEG-002

| Field | Complete record |
| --- | --- |
| Level | legs |
| Priority | MUST |
| Requirement statement | Untethered level-floor walking shall be the required HR-30W end-state and shall occur only after HR-30D slack-tether acceptance passes. |
| Verification ID | AUDIT-LEG-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Gate ordering is auditable; approval authority and anti-bypass implementation are undefined. |
| Evidence required to close | Implement signed gate/config inhibition and change-impact rules. |

## C.11 — Walking requirements

C.39 — WALK-001

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | HR-30W shall stand untethered for 120 seconds on the validated level indoor surface. |
| Verification ID | TEST-WALK-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 120 s is numeric; posture bounds, floor, no-step criterion, CoP/attitude limits and allowable corrections are incomplete. |
| Evidence required to close | Define scored state and all stability/thermal/deadline thresholds. |

C.40 — WALK-002

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | HR-30W shall start walk 25 m at 0.10 to 0.20 m/s stop within two steps and remain standing. |
| Verification ID | TEST-WALK-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | Distance/speed/stop steps are numeric; path width, step definition, startup/terminal bounds and failure taxonomy need procedure. |
| Evidence required to close | Release course geometry, instrumentation and exact stop/standing acceptance. |

C.41 — WALK-003

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | HR-30W shall turn 90 degrees left and right within five steps without support. |
| Verification ID | TEST-WALK-003 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | Angle/steps are numeric; turn radius, foot contact, corridor and overshoot are undefined. |
| Evidence required to close | Define geometry and measurement tolerances. |

C.42 — WALK-004

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | HR-30W shall complete 30 minutes cumulative walking without exceeding released thermal current voltage or energy limits. |
| Verification ID | TEST-WALK-004 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 30 min is numeric; ‘released limits’ are not yet selected and cumulative/rest timing is ambiguous. |
| Evidence required to close | Define continuous/cumulative schedule, cool-down rules and all numeric limits. |

C.43 — WALK-005

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | HR-30D shall walk with a slack overhead tether carrying less than 2 percent of robot weight except during fall arrest. |
| Verification ID | TEST-WALK-005 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 2% is numeric; tether load sensor accuracy/filtering/slack criterion and arrest exclusions need procedure. |
| Evidence required to close | Specify calibrated load cell, bandwidth, threshold duration and scored/arrest segmentation. |

C.44 — WALK-006

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | Each leg shall provide six active axes: hip yaw roll and pitch knee pitch and ankle pitch and roll. |
| Verification ID | INSPECT-WALK-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | Axis inventory is objective; range, lock state and effective DOF under coupling are not part of this requirement. |
| Evidence required to close | Add range/independence inspection tied to CAD. |

C.45 — WALK-007

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | Pelvis attitude estimation shall update at 800 Hz or faster and foot force/center-of-pressure estimation at 500 Hz or faster. |
| Verification ID | TEST-WALK-006 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | Rates are numeric; latency, jitter, synchronization, accuracy and invalid-data behavior are not fully specified. |
| Evidence required to close | Add end-to-end age/jitter/error thresholds and measurement procedure. |

C.46 — WALK-008

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | The real-time state estimator and stabilizer shall continue safe operation or stop after failure of the Linux high-level process. |
| Verification ID | TEST-WALK-007 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | ‘Continue safe operation or stop’ offers two outcomes without state-dependent threshold or time. |
| Evidence required to close | Define allowed response by state and maximum transition/stopping time. |

C.47 — WALK-009

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | HR-30W shall use an onboard professionally assembled protected battery with pack-level fuse service disconnect precharge and independent telemetry. |
| Verification ID | INSPECT-BATT-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Component classes are listed but no ratings, fault functions, charger interlock or pack acceptance thresholds. |
| Evidence required to close | Release battery safety specification and qualified inspection/electrical/abuse tests. |

C.48 — WALK-010

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | Passing the engineering walking test shall not authorize operation among people or children. |
| Verification ID | AUDIT-WALK-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Administrative boundary is auditable but not a performance requirement; authorization roles/site controls are unspecified. |
| Evidence required to close | Create operating release document and access-control audit. |

C.49 — WALK-011

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | A fall predictor shall command the released protective response from attitude angular rate support force joint tracking bus health and supply state. |
| Verification ID | TEST-FALL-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | ‘Released protective response’ is undefined and no detection time/false-negative/false-positive threshold exists. |
| Evidence required to close | Define fall classes, predictor horizon, thresholds and response per state. |

C.50 — WALK-012

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | A hardware emergency stop shall remove hazardous drive energy even when doing so may cause a fall. |
| Verification ID | TEST-SAFE-WALK-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Energy-removal intent is clear, but hazardous-energy threshold, response/stopping time and collapse consequences are unbounded. |
| Evidence required to close | Define safety function and restrained fault test by pose/mode. |

## C.12 — Stance requirements

C.51 — STANCE-001

| Field | Complete record |
| --- | --- |
| Level | stance |
| Priority | MUST |
| Requirement statement | During HR-30C acceptance both feet shall support 100 percent of measured static robot weight for 120 seconds while the connected restraint supplies no sustained support. |
| Verification ID | TEST-STANCE-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 120 s and 100% weight are numeric; ‘no sustained support’ needs tether-load threshold/bandwidth and posture bounds. |
| Evidence required to close | Define restraint-load threshold and standing state limits. |

C.52 — STANCE-002

| Field | Complete record |
| --- | --- |
| Level | stance |
| Priority | MUST |
| Requirement statement | HR-30C shall complete 100 commanded bilateral weight shifts with both feet in contact and without exceeding released force attitude joint-tracking thermal current or deadline limits. |
| Verification ID | TEST-STANCE-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | NUMERIC CORE, incomplete conditions |
| Can the method demonstrate the requirement? | Potentially capable after design/procedure closure |
| Evidence status | NONE — no executed approved record |
| Audit result | OBJECTIVE DRAFT — NO EVIDENCE |
| Exact gap | 100 shifts is numeric; force/CoP/attitude/tracking/thermal/deadline limits are all deferred. |
| Evidence required to close | Release numerical bounds and trajectory/test protocol. |

C.53 — STANCE-003

| Field | Complete record |
| --- | --- |
| Level | stance |
| Priority | MUST |
| Requirement statement | Loss of planner state sensor validity control deadline or supply health during HR-30C shall produce the released restrained response and a latched restart-required fault. |
| Verification ID | TEST-STANCE-003 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | ‘Released restrained response’ is circular; fault set, response time and safe state are undefined. |
| Evidence required to close | Split into fault-specific requirements and tests. |

C.54 — STANCE-004

| Field | Complete record |
| --- | --- |
| Level | stance |
| Priority | MUST |
| Requirement statement | HR-30C powered-stance testing shall not begin until HR-30B W0 W1 restraint proof sensor calibration and enabled-axis qualification records are approved. |
| Verification ID | AUDIT-STANCE-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Audit gate is sensible; record approval authority, version matching and non-bypass mechanism are absent. |
| Evidence required to close | Create signed prerequisite matrix and configuration lock. |

## C.13 — Product requirements

C.55 — PROD-011

| Field | Complete record |
| --- | --- |
| Level | product |
| Priority | MUST |
| Requirement statement | The released operating boundary shall state adult-operated experimental robotic machinery not a toy and not approved for unsupervised consumer use while legal classification remains unresolved. |
| Verification ID | AUDIT-PROD-002 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | QUALITATIVE |
| Can the method demonstrate the requirement? | Potentially capable after requirements refinement |
| Evidence status | NONE — no executed approved record |
| Audit result | DRAFT — NO EVIDENCE |
| Exact gap | Text audit is objective but legal classification remains open; it does not validate product safety. |
| Evidence required to close | Complete jurisdiction/intended-use review and retain exact operating boundary. |

## C.14 — Mechanical requirements

C.56 — MASS-001

| Field | Complete record |
| --- | --- |
| Level | mechanical |
| Priority | MUST |
| Requirement statement | Every full-body configuration shall have a controlled part-level mass and inertia ledger separating target estimate and measured values and reporting assembly and whole-body center of mass and inertia. |
| Verification ID | REVIEW-MASS-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Required ledger fields are conceptually adequate; accuracy, coordinate frames, measurement methods and reserve threshold are absent. |
| Evidence required to close | Release ledger schema, CAD/measured reconciliation and independent approval. |

## C.15 — Control requirements

C.57 — CTRL-005

| Field | Complete record |
| --- | --- |
| Level | control |
| Priority | MUST |
| Requirement statement | Every inter-controller message shall carry a monotonic sequence number source timestamp validity deadline sender state and configuration hash and receivers shall reject stale duplicate out-of-order or mismatched messages. |
| Verification ID | TEST-CTRL-005 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Message fields are listed; max latency/jitter/drop rate, clock basis and fault reaction are not numerical. |
| Evidence required to close | Define protocol/interface control document and timing/fault tests. |

## C.16 — Walking requirements

C.58 — WALK-013

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | Every externally reduced leg joint shall use output-side absolute position sensing or a reviewed equivalent that detects slip backlash lost calibration and motor-output disagreement. |
| Verification ID | TEST-JOINT-ENC-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Failure modes are named; accuracy, disagreement threshold, detection time, redundancy and calibration retention are not. |
| Evidence required to close | Select encoder and publish error/detection budget with injected-slip tests. |

C.59 — WALK-014

| Field | Complete record |
| --- | --- |
| Level | walking |
| Priority | MUST |
| Requirement statement | Each segmented joint bus shall demonstrate the released command and feedback schedule with a byte-level packet budget measured worst-case latency jitter utilization and injected fault traffic. |
| Verification ID | TEST-BUS-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Budget metrics are named but no pass thresholds for utilization, latency, jitter, retry traffic or missed frames. |
| Evidence required to close | Set nominal/fault utilization and deadline thresholds; measure on final harness. |

## C.17 — Safety requirements

C.60 — SAFE-009

| Field | Complete record |
| --- | --- |
| Level | safety |
| Priority | MUST |
| Requirement statement | Every powered full-body phase shall define and test the response to sudden drive-energy loss for every released pose and mode before restraint dependence is reduced. |
| Verification ID | TEST-POWERLOSS-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — gate/matrix concept only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | ‘Every pose and mode’ is not a finite test definition; safe outcome and energy-removal/collapse limits are missing. |
| Evidence required to close | Define finite safety-equivalence classes and pose-by-pose analysis/test matrix. |

## C.18 — Configuration requirements

C.61 — CFG-002

| Field | Complete record |
| --- | --- |
| Level | configuration |
| Priority | MUST |
| Requirement statement | A configuration calibration polarity or mirrored-joint mismatch shall inhibit drive enable and latch a recorded fault. |
| Verification ID | TEST-CFG-002 |
| Requirement status | draft |
| Objective verification method | NO — verification ID named only |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Mismatch fault intent is clear; configuration sources, comparison timing and fault thresholds are unspecified. |
| Evidence required to close | Define signed configuration schema, startup/runtime checks and injected mismatch tests. |

## C.19 — Verification requirements

C.62 — GOV-001

| Field | Complete record |
| --- | --- |
| Level | verification |
| Priority | MUST |
| Requirement statement | Every requirement risk and release gate shall identify an accountable owner acceptance evidence approver and controlled change history before design review approval. |
| Verification ID | AUDIT-GOV-001 |
| Requirement status | draft |
| Objective verification method | PARTIAL — method type named, no released procedure |
| Acceptance threshold | PARTIAL / circular / selection-dependent |
| Can the method demonstrate the requirement? | Not demonstrable as written |
| Evidence status | NONE — no executed approved record |
| Audit result | INSUFFICIENT / UNVERIFIABLE |
| Exact gap | Acceptance concepts are listed, but current schemas lack the required fields; audit cannot pass itself. |
| Evidence required to close | Add owner/rationale/threshold/evidence/approver/history fields and baseline governance process. |

# Appendix D — Complete Electrical Selection Closure Register

Record count: 106 unresolved selections. Procurement remains prohibited where the record says NO.

## D.1 — KiCad sheet 01_ac_pe_power

D.1 — JAC1: AC inlet

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | JAC1 |
| Item | AC inlet |
| Unresolved reason | Regional inlet, cord retention, ingress protection, and terminals are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.2 — Q0: Two-pole main disconnect

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | Q0 |
| Item | Two-pole main disconnect |
| Unresolved reason | Disconnect rating, poles, fault rating, and jurisdiction are unresolved. PE bypasses Q0. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.3 — QF1: PS1 AC branch protection

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | QF1 |
| Item | PS1 AC branch protection |
| Unresolved reason | Rating and curve require prospective fault current plus the unpublished LRS-350 inrush maximum, duration, waveform, T50, and I-squared-t. Typical inrush alone is insufficient. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.4 — QF2: PS2 AC branch protection

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | QF2 |
| Item | PS2 AC branch protection |
| Unresolved reason | Rating and curve require prospective fault current and inrush coordination. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.5 — QF3: PS3 AC branch protection

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | QF3 |
| Item | PS3 AC branch protection |
| Unresolved reason | Regional compute-supply branch protection and inlet arrangement are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.6 — PS3: Raspberry Pi 27 W USB-C supply

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | PS3 |
| Item | Raspberry Pi 27 W USB-C supply |
| Unresolved reason | Regional order code, receptacle, and AC integration are unresolved; USB-C is a functional interface, not a pin-level cable drawing. |
| Evidence required | Official Raspberry Pi product information; regional variant selection required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Official Raspberry Pi product information; regional variant selection required; inspection or functional test record. |

D.7 — PETB1: Protective-earth distribution block

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | PETB1 |
| Item | Protective-earth distribution block |
| Unresolved reason | Terminal block, bonding studs, conductor sizes, and continuity-test points are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.8 — SP1: Removable single-point DC 0 V / PE bond assembly

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 01_ac_pe_power |
| Reference | SP1 |
| Item | Removable single-point DC 0 V / PE bond assembly |
| Unresolved reason | DEFAULT OPEN. Populate only the single reviewed link; do not bridge all return domains by assumption. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.2 — KiCad sheet 02_hrv0_safety

D.9 — S0: Dual-NC positively-opening E-stop

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 02_hrv0_safety |
| Reference | S0 |
| Item | Dual-NC positively-opening E-stop |
| Unresolved reason | IDEC XW dual-NC is a family candidate only. Freeze the exact complete order code, operator/reset/contact/cover/environment details, attach the exact bottom-view drawing, and continuity-test the received unit before replacing TBD terminals. |
| Evidence required | IDEC XW candidate page and family catalogs reviewed 2026-08-05; exact order code and device drawing remain SELECTION REQUIRED |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-03 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; IDEC XW candidate page and family catalogs reviewed 2026-08-05; exact order code and device drawing remain SELECTION REQUIRED; inspection or functional test record. |

D.10 — S1: Momentary NO reset pushbutton

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 02_hrv0_safety |
| Reference | S1 |
| Item | Momentary NO reset pushbutton |
| Unresolved reason | Exact normally-open contact and terminal identifiers are unresolved. Locate outside swept envelope. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-03 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.11 — FSR1: PNOZ output-contact protection

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 02_hrv0_safety |
| Reference | FSR1 |
| Item | PNOZ output-contact protection |
| Unresolved reason | Pilz manual p.11 requires output-contact protection. Published maxima are not selections; coordinate the exact protection with coil inrush, conductor, PS2 behavior, and fault current. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-03 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.12 — SK1: K1 DC-coil suppression network

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 02_hrv0_safety |
| Reference | SK1 |
| Item | K1 DC-coil suppression network |
| Unresolved reason | Select per Pilz and Schneider instructions. Exact device, polarity, energy rating, fault behavior, and effect on dropout/stopping time require review and test. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-03 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.13 — SK2: K2 DC-coil suppression network

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 02_hrv0_safety |
| Reference | SK2 |
| Item | K2 DC-coil suppression network |
| Unresolved reason | Select per Pilz and Schneider instructions. Exact device, polarity, energy rating, fault behavior, and effect on dropout/stopping time require review and test. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-03 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.3 — KiCad sheet 03_hrv0_distribution

D.14 — F0: HR-V0 main DC fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 03_hrv0_distribution |
| Reference | F0 |
| Item | HR-V0 main DC fuse |
| Unresolved reason | Rating, holder, interrupt rating, conductor protection, and clearing coordination are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.15 — F1: Shoulder branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 03_hrv0_distribution |
| Reference | F1 |
| Item | Shoulder branch fuse |
| Unresolved reason | No fuse value is released. Requires measured current, inrush, cable, connector, ambient, and duty-cycle data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.16 — F2: Elbow branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 03_hrv0_distribution |
| Reference | F2 |
| Item | Elbow branch fuse |
| Unresolved reason | No fuse value is released. Requires measured current, inrush, cable, connector, ambient, and duty-cycle data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.17 — F3: Gripper branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 03_hrv0_distribution |
| Reference | F3 |
| Item | Gripper branch fuse |
| Unresolved reason | No fuse value is released. Requires measured current, inrush, cable, connector, ambient, and duty-cycle data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.4 — KiCad sheet 04_control_watchdog

D.18 — PI1: Raspberry Pi 5 8GB high-level compute

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 04_control_watchdog |
| Reference | PI1 |
| Item | Raspberry Pi 5 8GB high-level compute |
| Unresolved reason | Exact port allocation, GPIO interface board, grounding, and cable retention are unresolved. |
| Evidence required | Official Raspberry Pi product page; board-level integration and connector assignments remain open |
| Review priority | BLOCKER |
| Release gate | HR-V0 + HR-30 functional safety / G-03,G-16 |
| Responsible discipline | Controls + electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Official Raspberry Pi product page; board-level integration and connector assignments remain open; inspection or functional test record. |

D.19 — RT1: STM32H743-class deterministic motion controller

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 04_control_watchdog |
| Reference | RT1 |
| Item | STM32H743-class deterministic motion controller |
| Unresolved reason | Exact controller board, transceivers, pin map, isolation, and power integrity are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 + HR-30 functional safety / G-03,G-16 |
| Responsible discipline | Controls + electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.20 — WD1: Independent HR-V0 watchdog controller

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 04_control_watchdog |
| Reference | WD1 |
| Item | Independent HR-V0 watchdog controller |
| Unresolved reason | Exact RP2040-class board, brownout response, firmware integrity, and startup state are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 + HR-30 functional safety / G-03,G-16 |
| Responsible discipline | Controls + electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.21 — QWD1: HR-V0 3.3 V-to-24 V watchdog coil driver

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 04_control_watchdog |
| Reference | QWD1 |
| Item | HR-V0 3.3 V-to-24 V watchdog coil driver |
| Unresolved reason | Transistor or optocoupler circuit, logic polarity, flyback coordination, creepage, and failure response are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 + HR-30 functional safety / G-03,G-16 |
| Responsible discipline | Controls + electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.22 — WDH1: Independent HR-30 watchdog controller

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 04_control_watchdog |
| Reference | WDH1 |
| Item | Independent HR-30 watchdog controller |
| Unresolved reason | Separate variant hardware. Exact controller, startup state, brownout, and fault latch are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 + HR-30 functional safety / G-03,G-16 |
| Responsible discipline | Controls + electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.23 — QWDH1: HR-30 3.3 V-to-24 V watchdog coil driver

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 04_control_watchdog |
| Reference | QWDH1 |
| Item | HR-30 3.3 V-to-24 V watchdog coil driver |
| Unresolved reason | Separate variant driver. Exact circuit and fault response are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 + HR-30 functional safety / G-03,G-16 |
| Responsible discipline | Controls + electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.24 — ISO1: Isolated safety-status input module

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 04_control_watchdog |
| Reference | ISO1 |
| Item | Isolated safety-status input module |
| Unresolved reason | Input thresholds, isolation ratings, diagnostics, and exact terminals are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-V0 + HR-30 functional safety / G-03,G-16 |
| Responsible discipline | Controls + electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.5 — KiCad sheet 05_hr30_battery_precharge

D.25 — BAT1: Professionally assembled 4-series protected battery pack

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | BAT1 |
| Item | Professionally assembled 4-series protected battery pack |
| Unresolved reason | Qualified supplier, cells, configuration, protected ports, pack limits, current, capacity, enclosure, and telemetry are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.26 — FBAT1: Pack-level fuse at battery

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | FBAT1 |
| Item | Pack-level fuse at battery |
| Unresolved reason | Rating and interrupt capability require pack fault-current data and conductor protection calculation. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.27 — SD1: Accessible DC service disconnect

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | SD1 |
| Item | Accessible DC service disconnect |
| Unresolved reason | Exact DC rating, poles, breaking capacity, touch protection, and lockout method are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.28 — KB1: Battery main source contactor

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | KB1 |
| Item | Battery main source contactor |
| Unresolved reason | Coil voltage, DC make/break duty, inrush, mirror contact, suppression, and fault coordination are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.29 — KP1: Precharge relay

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | KP1 |
| Item | Precharge relay |
| Unresolved reason | Contact voltage/current, coil drive, welded-contact detection, and timing are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.30 — RP1: Precharge resistor

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | RP1 |
| Item | Precharge resistor |
| Unresolved reason | Resistance, pulse energy, power, tolerance, mounting, temperature, and discharge interval require measured load capacitance. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.31 — RD1: DC-link discharge path

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | RD1 |
| Item | DC-link discharge path |
| Unresolved reason | Discharge resistance, power, touch-safe voltage time, and thermal mounting are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.32 — JCHG1: Charge inlet

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | JCHG1 |
| Item | Charge inlet |
| Unresolved reason | Connector, keying, touch protection, current, voltage, and presence-contact implementation are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.33 — CHG1: Battery charger / protected charge interface

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | CHG1 |
| Item | Battery charger / protected charge interface |
| Unresolved reason | Charger compatibility, BMS charge-port topology, communications, isolation, and interlock circuit are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.34 — TLM1: Independent battery telemetry interface

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | TLM1 |
| Item | Independent battery telemetry interface |
| Unresolved reason | Isolation, voltage measurement, current sensing, accuracy, fault behavior, and exact interface are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.35 — BCU1: Independent battery sequencing and interlock controller

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 05_hr30_battery_precharge |
| Reference | BCU1 |
| Item | Independent battery sequencing and interlock controller |
| Unresolved reason | Controller, independent power, voltage threshold circuit, coil drivers, diagnostics, sequencing logic, and safety-inhibit interface are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30W / G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.6 — KiCad sheet 06_hr30_safety_distribution

D.36 — JT1: HR-30 tethered ELV source inlet

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | JT1 |
| Item | HR-30 tethered ELV source inlet |
| Unresolved reason | 14.0-14.8 V current-limited 80 A-class source, connector, cable, PE/shield policy, and ratings are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.37 — FTHR1: Tether source protection

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | FTHR1 |
| Item | Tether source protection |
| Unresolved reason | Rating and protection coordination require source fault-current and cable data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.38 — SRCSEL1: Mutually exclusive tether/battery source selector

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | SRCSEL1 |
| Item | Mutually exclusive tether/battery source selector |
| Unresolved reason | Break-before-make or variant-keyed implementation is unresolved. Sources must never be electrically paralleled. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.39 — FHR0: HR-30 main actuator fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | FHR0 |
| Item | HR-30 main actuator fuse |
| Unresolved reason | Rating, holder, interrupt capacity, cable protection, and source coordination are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.40 — SH0: HR-30 dual-NC positively-opening E-stop

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | SH0 |
| Item | HR-30 dual-NC positively-opening E-stop |
| Unresolved reason | IDEC XW dual-NC is a family candidate only. Freeze the exact complete order code, operator/reset/contact/cover/environment details, attach the exact bottom-view drawing, and continuity-test the received unit before replacing TBD terminals. |
| Evidence required | IDEC XW candidate page and family catalogs reviewed 2026-08-05; exact order code and device drawing remain SELECTION REQUIRED |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; IDEC XW candidate page and family catalogs reviewed 2026-08-05; exact order code and device drawing remain SELECTION REQUIRED; inspection or functional test record. |

D.41 — SH1: HR-30 momentary NO reset pushbutton

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | SH1 |
| Item | HR-30 momentary NO reset pushbutton |
| Unresolved reason | Exact normally-open contact, terminal identifiers, location, and environmental rating are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.42 — FSRH1: HR-30 PNOZ output-contact protection

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | FSRH1 |
| Item | HR-30 PNOZ output-contact protection |
| Unresolved reason | Pilz manual p.11 requires output-contact protection. Published maxima are not selections; coordinate the exact protection with coil inrush, conductor, PS2 behavior, and fault current. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.43 — KH1: HR-30 actuator-power contactor 1

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | KH1 |
| Item | HR-30 actuator-power contactor 1 |
| Unresolved reason | DC-rated 50 A-class architecture input only. Exact device, mirror-contact validity, coil, DC duty, inrush, and protection coordination are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.44 — KH2: HR-30 actuator-power contactor 2

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | KH2 |
| Item | HR-30 actuator-power contactor 2 |
| Unresolved reason | Second independent series contactor. Exact device and all coordination evidence are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.45 — SKH1: KH1 DC-coil suppression network

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | SKH1 |
| Item | KH1 DC-coil suppression network |
| Unresolved reason | Select per Pilz and chosen contactor instructions. Exact device, polarity, energy rating, fault behavior, and effect on dropout/stopping time require review and test. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.46 — SKH2: KH2 DC-coil suppression network

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | SKH2 |
| Item | KH2 DC-coil suppression network |
| Unresolved reason | Select per Pilz and chosen contactor instructions. Exact device, polarity, energy rating, fault behavior, and effect on dropout/stopping time require review and test. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.47 — DC1: HR-30 upper-body regulated 12 V converter

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 06_hr30_safety_distribution |
| Reference | DC1 |
| Item | HR-30 upper-body regulated 12 V converter |
| Unresolved reason | Required because the HR-30 source is not assumed to be a verified 12 V rail. Power, efficiency, isolation, transient, and thermal ratings are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | HR-30A-C/D/W / G-12,G-17,G-23 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.7 — KiCad sheet 07_left_leg_rs485

D.48 — XCVRL1: left leg isolated RS-485 transceiver

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | XCVRL1 |
| Item | left leg isolated RS-485 transceiver |
| Unresolved reason | Exact transceiver, isolation, fail-safe bias, common-mode range, ESD protection, connector, and controller pins are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.49 — RTL1: left leg near-end RS-485 termination

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | RTL1 |
| Item | left leg near-end RS-485 termination |
| Unresolved reason | Fit only at the verified controller-side physical end. Resistance and placement require cable-impedance and transceiver review. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.50 — RTL2: left leg far-end RS-485 termination

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | RTL2 |
| Item | left leg far-end RS-485 termination |
| Unresolved reason | Fit only at the verified far physical end. No intermediate termination is permitted. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.51 — FL1: left hip yaw actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | FL1 |
| Item | left hip yaw actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.52 — FL2: left hip roll actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | FL2 |
| Item | left hip roll actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.53 — FL3: left hip pitch actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | FL3 |
| Item | left hip pitch actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.54 — FL4: left knee pitch actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | FL4 |
| Item | left knee pitch actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.55 — FL5: left ankle pitch actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | FL5 |
| Item | left ankle pitch actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.56 — FL6: left ankle roll actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 07_left_leg_rs485 |
| Reference | FL6 |
| Item | left ankle roll actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.8 — KiCad sheet 08_right_leg_rs485

D.57 — XCVRR1: right leg isolated RS-485 transceiver

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | XCVRR1 |
| Item | right leg isolated RS-485 transceiver |
| Unresolved reason | Exact transceiver, isolation, fail-safe bias, common-mode range, ESD protection, connector, and controller pins are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.58 — RTR1: right leg near-end RS-485 termination

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | RTR1 |
| Item | right leg near-end RS-485 termination |
| Unresolved reason | Fit only at the verified controller-side physical end. Resistance and placement require cable-impedance and transceiver review. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.59 — RTR2: right leg far-end RS-485 termination

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | RTR2 |
| Item | right leg far-end RS-485 termination |
| Unresolved reason | Fit only at the verified far physical end. No intermediate termination is permitted. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.60 — FR1: right hip yaw actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | FR1 |
| Item | right hip yaw actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.61 — FR2: right hip roll actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | FR2 |
| Item | right hip roll actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.62 — FR3: right hip pitch actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | FR3 |
| Item | right hip pitch actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.63 — FR4: right knee pitch actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | FR4 |
| Item | right knee pitch actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.64 — FR5: right ankle pitch actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | FR5 |
| Item | right ankle pitch actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.65 — FR6: right ankle roll actuator branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 08_right_leg_rs485 |
| Reference | FR6 |
| Item | right ankle roll actuator branch fuse |
| Unresolved reason | Individual branch protection required by ELEC-003. Rating and holder require measured current, cable, connector, ambient, bundling, and fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30B/C/D/W / G-14,G-22 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.9 — KiCad sheet 09_upper_bus_a

D.66 — FU31: left shoulder pitch branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU31 |
| Item | left shoulder pitch branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.67 — FU32: left shoulder roll branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU32 |
| Item | left shoulder roll branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.68 — FU33: left elbow branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU33 |
| Item | left elbow branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.69 — FU34: left wrist branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU34 |
| Item | left wrist branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.70 — FU35: left gripper branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU35 |
| Item | left gripper branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.71 — FU51: waist yaw branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU51 |
| Item | waist yaw branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.72 — FU52: head pan branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU52 |
| Item | head pan branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.73 — FU53: head tilt branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | FU53 |
| Item | head tilt branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.10 — KiCad sheet 10_upper_bus_b

D.74 — FU41: right shoulder pitch branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 10_upper_bus_b |
| Reference | FU41 |
| Item | right shoulder pitch branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.75 — FU42: right shoulder roll branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 10_upper_bus_b |
| Reference | FU42 |
| Item | right shoulder roll branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.76 — FU43: right elbow branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 10_upper_bus_b |
| Reference | FU43 |
| Item | right elbow branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.77 — FU44: right wrist branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 10_upper_bus_b |
| Reference | FU44 |
| Item | right wrist branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.78 — FU45: right gripper branch fuse

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 10_upper_bus_b |
| Reference | FU45 |
| Item | right gripper branch fuse |
| Unresolved reason | Individual fuse and holder rating require the released current profile, connector, cable, ambient, bundling, and converter fault data. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.11 — KiCad sheet 11_sensors_sync

D.79 — LCL1: left foot 1 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCL1 |
| Item | left foot 1 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.80 — LCL2: left foot 2 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCL2 |
| Item | left foot 2 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.81 — LCL3: left foot 3 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCL3 |
| Item | left foot 3 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.82 — LCL4: left foot 4 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCL4 |
| Item | left foot 4 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.83 — LCR1: right foot 1 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCR1 |
| Item | right foot 1 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.84 — LCR2: right foot 2 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCR2 |
| Item | right foot 2 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.85 — LCR3: right foot 3 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCR3 |
| Item | right foot 3 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.86 — LCR4: right foot 4 compression load cell 0-200 N

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 11_sensors_sync |
| Reference | LCR4 |
| Item | right foot 4 compression load cell 0-200 N |
| Unresolved reason | Exact load cell, bridge wiring, overload rating, cable, connector, calibration, and cross-axis behavior are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.12 — KiCad sheet 12_sensor_adc_imu

D.87 — ADC1: TI ADS131M04 left foot ADC - proposed device

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 12_sensor_adc_imu |
| Reference | ADC1 |
| Item | TI ADS131M04 left foot ADC - proposed device |
| Unresolved reason | Exact package, pin map, clock, reference, analog power, protection, anti-alias network, and PCB layout are unresolved. |
| Evidence required | TI ADS131M04 datasheet Rev D, May 2021; functional net names only until package selection |
| Review priority | BLOCKER |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; TI ADS131M04 datasheet Rev D, May 2021; functional net names only until package selection; inspection or functional test record. |

D.88 — ADC2: TI ADS131M04 right foot ADC - proposed device

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 12_sensor_adc_imu |
| Reference | ADC2 |
| Item | TI ADS131M04 right foot ADC - proposed device |
| Unresolved reason | Exact package, pin map, clock, reference, analog power, protection, anti-alias network, and PCB layout are unresolved. |
| Evidence required | TI ADS131M04 datasheet Rev D, May 2021; functional net names only until package selection |
| Review priority | BLOCKER |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; TI ADS131M04 datasheet Rev D, May 2021; functional net names only until package selection; inspection or functional test record. |

D.89 — IMU1: Bosch BMI088 pelvis IMU - proposed device

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 12_sensor_adc_imu |
| Reference | IMU1 |
| Item | Bosch BMI088 pelvis IMU - proposed device |
| Unresolved reason | Exact package pins, decoupling, separate accel/gyro interfaces, PCB layout, clocking, and mechanical alignment are unresolved. |
| Evidence required | Bosch BMI088 datasheet Rev 1.9, January 2024; functional interface only |
| Review priority | BLOCKER |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Bosch BMI088 datasheet Rev 1.9, January 2024; functional interface only; inspection or functional test record. |

D.90 — IMU2: Bosch BMI088 torso IMU - proposed device

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 12_sensor_adc_imu |
| Reference | IMU2 |
| Item | Bosch BMI088 torso IMU - proposed device |
| Unresolved reason | Exact package pins, decoupling, PCB layout, clocking, and mechanical alignment are unresolved. |
| Evidence required | Bosch BMI088 datasheet Rev 1.9, January 2024; functional interface only |
| Review priority | BLOCKER |
| Release gate | HR-30C/D/W / G-15,G-16 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Bosch BMI088 datasheet Rev 1.9, January 2024; functional interface only; inspection or functional test record. |

## D.13 — KiCad sheet 13_sensor_afe

D.91 — AFE1: Load-cell excitation, reference, protection, and anti-alias front ends

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 13_sensor_afe |
| Reference | AFE1 |
| Item | Load-cell excitation, reference, protection, and anti-alias front ends |
| Unresolved reason | Excitation voltage, reference, amplifiers if required, filters, protection, calibration injection, and complete PCB circuit are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | HR-30C/D/W / G-15 |
| Responsible discipline | Electrical + controls + test |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.14 — KiCad sheet 14_harness_terminals

D.92 — TBAC1: Guarded mains terminal block

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | TBAC1 |
| Item | Guarded mains terminal block |
| Unresolved reason | No exposed mains terminal may be accessible without a tool. Terminal family, jumpers, covers, and ratings are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.93 — TB24: 24 V safety-control terminal block

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | TB24 |
| Item | 24 V safety-control terminal block |
| Unresolved reason | This block is downstream of PS2 only; it has no electrical connection to TBAC1 beyond the power-supply isolation boundary. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.94 — TBV0: HR-V0 actuator terminal block

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | TBV0 |
| Item | HR-V0 actuator terminal block |
| Unresolved reason | Terminal selection and conductor allocation are unresolved. No terminal on TBV0 feeds HR-30. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.95 — TBHR30: HR-30 actuator terminal block

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | TBHR30 |
| Item | HR-30 actuator terminal block |
| Unresolved reason | Terminal selection, branch allocation, current sharing, barriers, and conductor sizing are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.96 — JHL1: Left-leg harness bulkhead

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | JHL1 |
| Item | Left-leg harness bulkhead |
| Unresolved reason | Connector family, pin numbers, current per contact, keying, shielding, strain relief, and ingress rating are unresolved. Prohibit unreviewed full-conductor DYNAMIXEL links; release a VDD-isolating data harness and require continuity/no-backfeed tests. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.97 — JHR1: Right-leg harness bulkhead

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | JHR1 |
| Item | Right-leg harness bulkhead |
| Unresolved reason | Connector family, pin numbers, current per contact, keying, shielding, strain relief, and ingress rating are unresolved. Prohibit unreviewed full-conductor DYNAMIXEL links; release a VDD-isolating data harness and require continuity/no-backfeed tests. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.98 — JUA1: Upper-body bus A harness

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | JUA1 |
| Item | Upper-body bus A harness |
| Unresolved reason | Connector family, pin map, current capacity, contact count, keying, and strain relief are unresolved. Prohibit standard full-conductor TTL daisy cables; release a data-only harness and require continuity/no-backfeed tests. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.99 — JUB1: Upper-body bus B harness

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | JUB1 |
| Item | Upper-body bus B harness |
| Unresolved reason | Connector family, pin map, current capacity, contact count, keying, and strain relief are unresolved. Prohibit standard full-conductor TTL daisy cables; release a data-only harness and require continuity/no-backfeed tests. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.100 — JSL1: Left-foot sensor harness

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | JSL1 |
| Item | Left-foot sensor harness |
| Unresolved reason | Cable topology, connector, differential/analog routing, shielding, and pin map are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.101 — JSR1: Right-foot sensor harness

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | JSR1 |
| Item | Right-foot sensor harness |
| Unresolved reason | Cable topology, connector, differential/analog routing, shielding, and pin map are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.102 — SHLD1: RS-485 shield-drain termination assembly

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | SHLD1 |
| Item | RS-485 shield-drain termination assembly |
| Unresolved reason | Single-ended, capacitive, or direct chassis termination policy requires EMC testing; do not create an unintended DC 0 V/PE bond. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | BLOCKER |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

D.103 — TP1: Commissioning test-point and lockout panel

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 14_harness_terminals |
| Reference | TP1 |
| Item | Commissioning test-point and lockout panel |
| Unresolved reason | Finger-safe test points, covers, labels, voltage categories, and lockout procedures are unresolved. |
| Evidence required | Manufacturer datasheet and engineering inputs required |
| Review priority | MAJOR |
| Release gate | All electrical releases / G-03,G-04,G-12,G-22 |
| Responsible discipline | Electrical + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Manufacturer datasheet and engineering inputs required; inspection or functional test record. |

## D.15 — KiCad sheet 03_hrv0_distribution

D.104 — UV1:2: U2D2 pin-2 VDD cable treatment

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 03_hrv0_distribution |
| Reference | UV1:2 |
| Item | U2D2 pin-2 VDD cable treatment |
| Unresolved reason | ROBOTIS does not document pin 2 as sense-only or publish the internal current path. |
| Evidence required | Written ROBOTIS evidence or a released custom data cable omitting pin 2; continuity and no-backfeed test record |
| Review priority | BLOCKER |
| Release gate | HR-V0 energization / G-04 |
| Responsible discipline | Electrical + functional safety |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Written ROBOTIS evidence or a released custom data cable omitting pin 2; continuity and no-backfeed test record; inspection or functional test record. |

## D.16 — KiCad sheet 09_upper_bus_a

D.105 — UA1:2: U2D2 pin-2 VDD cable treatment

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 09_upper_bus_a |
| Reference | UA1:2 |
| Item | U2D2 pin-2 VDD cable treatment |
| Unresolved reason | ROBOTIS does not document pin 2 as sense-only or publish the internal current path. |
| Evidence required | Written ROBOTIS evidence or a released custom data cable omitting pin 2; continuity and no-backfeed test record |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Written ROBOTIS evidence or a released custom data cable omitting pin 2; continuity and no-backfeed test record; inspection or functional test record. |

## D.17 — KiCad sheet 10_upper_bus_b

D.106 — UB1:2: U2D2 pin-2 VDD cable treatment

| Field | Complete record |
| --- | --- |
| Package status | PRELIMINARY - NOT APPROVED FOR ENERGIZATION |
| KiCad sheet | 10_upper_bus_b |
| Reference | UB1:2 |
| Item | U2D2 pin-2 VDD cable treatment |
| Unresolved reason | ROBOTIS does not document pin 2 as sense-only or publish the internal current path. |
| Evidence required | Written ROBOTIS evidence or a released custom data cable omitting pin 2; continuity and no-backfeed test record |
| Review priority | MAJOR |
| Release gate | HR-30A / G-12 |
| Responsible discipline | Electrical + controls + mechanical harness |
| Procurement allowed | NO |
| Closure record required | Exact MPN and pin map; rating/error-budget calculation; released schematic/harness/BOM revision; Written ROBOTIS evidence or a released custom data cable omitting pin 2; continuity and no-backfeed test record; inspection or functional test record. |

# Appendix E — Source Inventory and Integrity Record

The following source artifacts were consolidated into this dossier. SHA-256 hashes permit later confirmation that no row set was silently changed during assembly.

E.1 — Project_Button_Independent_Engineering_Review_HR-30-SYS-R0.2.md

| Field | Complete record |
| --- | --- |
| Data records | Narrative report |
| Size in bytes | 72619 |
| SHA-256 | 38610c92aab287e41fed1ca143ada83f3a557cb456b84c93a393cad0ea52328f |

E.2 — Project_Button_Finding_Register.csv

| Field | Complete record |
| --- | --- |
| Data records | 56 |
| Size in bytes | 34938 |
| SHA-256 | 5b5d940d97adef4e6f339f7870408330e38f3766b9a3c2f7b783babce6211f0b |

E.3 — Project_Button_Independent_Calculations.csv

| Field | Complete record |
| --- | --- |
| Data records | 40 |
| Size in bytes | 3892 |
| SHA-256 | 75acb88191b789e90207c9e0a58fba4514b69f77f3a6d32e6be82c1d8cc1860e |

E.4 — Project_Button_Requirement_Audit.csv

| Field | Complete record |
| --- | --- |
| Data records | 62 |
| Size in bytes | 32881 |
| SHA-256 | dbe3b15b2f8568b271af145799d6cb40e72a76aeb6f3440e8e8b0ce7bb826802 |

E.5 — Project_Button_Electrical_Selection_Closure.csv

| Field | Complete record |
| --- | --- |
| Data records | 106 |
| Size in bytes | 59521 |
| SHA-256 | eeed6c01ba1a89332a06f36fc12eed7a79cf67ecb38bea0b77ff87da3de02b4a |

## Consolidation statement

The narrative report and all four companion registers are embedded in this file. This consolidation changes presentation only; it does not close, downgrade or approve any finding, requirement, selection or release gate.